Customer Product Specification

SUZUKI YH Series ABS/ESC

Diagnostic Design Specification

CONTENTS

Changes	4

Previous Editions	4

1	SCOPE	4

2	AREA OF APPLICATION	4

3	REFERENCES	4

4	ABBREVIATION	5

5	CAN COMMUNICATION PROTOCOL	6

5.1	CAN message	6

5.2	Message transmission	6

5.3	Message frame structure	6

5.4	Communication timing parameter	6

6	DIAGNOSTIC MODE AND SERVICE	6

6.1	Diagnostic mode	6

6.2	Supported Service	6

7	DIAGNOSTIC SERVICE	6

7.1	Start Diagnostic Session (SID$10)	6

7.2	Clear Diagnostic Information (SID$14)	6

7.3	Read DTC By Status (SID$18)	6

7.3.1	Read DTC by Status Mask	6

7.4	Read ECU ID (SID: $1A)	6

7.5	Read Data By Local Identifier (SID$21)	6

7.6	Security Access (SID$27)	6

7.6.1	Seed Request (AM=$01)	6

7.6.2	Send Key Request (AM=$02)	6

7.7	Input Output Control By Local Identifier (SID$30)	6

7.7.1	Request Valve drive$00 $07	6

7.7.7.1	Air bleeding drive (secondary system)	6

7.7.7.2	Air bleeding drive (thirdly system)	6

7.7.7.3	Pressure decreasing drive	6

7.7.7.4	Pressure increasing drive	6

7.7.2	Request Sensor calibration $01 $08	6

7.7.7.1	Sensor calibration	6

7.7.3	Request Function settings $02 $07	6

7.7.7.1	ESP function disable - vehicle inspection mode	6

7.7.4	Request Various drive $03 $07	6

7.7.7.1	ECU force reset	6

7.7.5	Request Various drive 2 $04 $07	6

7.7.7.1	Brake Lamp	6

7.7.6	Request Fill and bleed $80 $07	6

7.7.7.1	Fill and bleed	6

7.7.7	Request Security $81 $07	6

7.7.7.1	Password verification	6

7.8	Write Data By Local Identifier (SID$3B)	6

7.8.1	Fell and bleed related flags writing RLI$80	6

7.8.2	variant registration RLI$81	6

7.8.3	RLI$98, 99	6

7.9	Tester present (SID$3E)	6

8	RLI stored Data Table	6

9	DTC List	6

10	Freeze Frame Data Specification	6

11	Drive recorder Specification	6

12	Flag – DTC Transition Table	6

13	Variant Coding Specification	6

14	CHANGE HISTORY	6

Changes

| Index | Date | Chapter | Contents |
| --- | --- | --- | --- |
| 00 |  | All | Newly created Based on YS Series |
| 01 |  | 7.5 | Support RLI$30, 31 |
| 01 |  | 8 | Add to *5) of RLI $00 Steering angle values(Position 25 and 26) |
| 01 |  | 8 | Add to Bit0 of Position 39 of RLI $00 Variant Coding Mismatch Detection status |

Previous Editions

None

SCOPE

This customer product specification applies to the diagnostic functionality for Suzuki YHA MK100 ABS/ESC units within Continental Automotive Corporation.

AREA OF APPLICATION

This customer product specification provides how to access the unit via using diagnostic communication.

REFERENCES

This specification is based on the following diagnostic specifications that offered from the customer.

SUZUKI SDL on CAN-KWP2000 SPECIFICATION PART1

SUZUKI SDL on CAN-KWP2000 SPECIFICATION PART2

ABBREVIATION

| Abbreviation | Meaning |
| --- | --- |
| ABS | Antilock Brake System |
| BLS | Brake Lamp Switch |
| CAC | Continental Automotive Corporation |
| CAN | Controller Area Network |
| DTC | Diagnostic Trouble Code |
| EBS | Electronic Brake System |
| ECU | Electronic Controller Unit |
| EEPROM | Electronically Erasable and Programmable Read Only Memory |
| FFD | Freeze Frame Data |
| HW | Hardware |
| IGN | Ignition |
| NRC | Negative Response Code |
| SID | Service Identifier |
| SW | Software |
| SWAS | Steering Wheel Angle Sensor |
| WL | Warning Lamp |

CAN COMMUNICATION PROTOCOL

This section describes about the diagnostic communication (ID, frame and timing etc.).

Note) The bit rate of CAN communication is 500kbit/s.

CAN message

The following messages are used for the diagnostic communication.

| Direction | CAN ID (hex) | Length (Byte) | Period | Note | Com. method | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Tester -> ECU | 261 | 8 | Not periodic | Physical Addressing | Tester : ECU = 1: 1 | 1. Support both Single and Multi- frame 2. ECU responds to the request. (Not respond if no response required) |
| ECU -> Tester | 661 | 8 | Not periodic |  |  |  |

Message transmission

According to the length of diagnostic message, the tester and ECU apply Single/Multi frame.

- Single frame: data length <= 7 bytes (i.e. Transmission finishes with 1 frame)

- Multi frame: data length > 7 bytes (i.e. Transmission doesn’t finish with 1 frame)

<Example for Multi frame>

Message frame structure

The message frame structure is the following table.

- Frame structure

| N_PDU type | CAN ID | CAN frame data field |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | byte0 | byte1 | byte2 | byte3 | byte4 | byte5 | byte6 | byte7 |
| Single Frame (SF) | N_AI | N_PCI | N_Data |  |  |  |  |  |  |
| First Frame (FF) | N_AI | N_PCI |  | N_Data |  |  |  |  |  |
| Consecutive Frame (CF) | N_AI | N_PCI | N_Data |  |  |  |  |  |  |
| Flow Control (FC) | N_AI | N_PCI |  |  | N/A (fixed to $00) |  |  |  |  |

- Description of each frame

| Name | Description |
| --- | --- |
| N_AI: Address Information | Message ID to transmit/receive 11bit CAN ID is used. |
| N_PCI: Protocol Control Information | High nibble (4-7bit) indicates the message type, and Low nibble (0-3bit) depends on the message. (See PCI structure below) |
| N_Data: Data Field | Data of diagnostic message. The vacancy byte is filled with $00. |

- PCI structure

| Message Type | N_PCI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | byte0 |  |  |  |  |  |  |  | byte1 |  |  |  |  |  |  |  | byte2 |  |  |  |  |  |  |  |
| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Single Frame (SF) | 0 | 0 | 0 | 0 | SF_DL |  |  |  | N_Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Frame (FF) | 0 | 0 | 0 | 1 | FF_DL |  |  |  |  |  |  |  |  |  |  |  | N_Data |  |  |  |  |  |  |  |
| Consecutive Frame (CF) | 0 | 0 | 1 | 0 | SN |  |  |  | N_Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flow Control (FC) | 0 | 0 | 1 | 1 | FS |  |  |  | BS |  |  |  |  |  |  |  | STmin |  |  |  |  |  |  |  |

- Detail of PCI

| Name (Frame) | Description |
| --- | --- |
| SF_DL (SF) | Data length for Single Frame (length: 0 ~ 7 byte) |
| FF_DL (FF) | Data length for Multi Frame (length: 8 ~ 4095 byte) |
| SN (CF) | Sequence number (0 <= SN <= 15). This value is 0 for First Frame. Therefore, 1 is set to 1st Consecutive Frame. This value for the next frame of 15th Consecutive Frame is 0. |
| FS (FC) | There are 3 types that CTS (start to transmit), WT (wait for transmission until CTS transmitted) and OVFLW (overflow of received buffer) 0000 (CTS): The receiver will receive Consecutive Frame with BS and STmin. 0001 (WT): The sender waits for next Flow Control from the receiver. 0010 (OVFLW): The sender stops to send Consecutive Frame. |
| BS: Block Size (FC) | The maximum number of receivable Consecutive Frame at once. The sender node shall send Consecutive Frame within Block Size. (If the sender node still has Consecutive Frame, the receiver node sends Flow Control Frame before the sender node sends the remained Consecutive Frame except for the case of that BS is 0.) |
| STmin: Separation Time (FC) | The minimum time of the interval between Consecutive Frames (msec). The sender node shall send Consecutive Frames with the interval more than STmin. |

Communication timing parameter

The communication timing is defined as the following.

- Network Layer

| Parameter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sender | Receiver | Description | Time-out [ms] | Recommended Value |
| N_As | --- | Transmission time Time between the start to send a message and complete it by sender node | 25 | --- |
| N_Bs | --- | Reply Waiting time Time between the complete to send a message and receive a next message by sender node | 75 | --- |
| N_Cs | --- | Transmission preparation time Time between the complete to receive/send a message and start to send a next message by sender node | --- | < 50ms |
| --- | N_Ar | Reply time Time between the start to send FC and complete it by receiver node | 25 | --- |
| --- | N_Br | Reply Reception time Time between the complete to receive/send a message and start to send a next message by receiver node | --- | < 25ms |
| --- | N_Cr | Transmission Waiting time Time between the complete to send/receive a message and complete to receive a message by receiver node | 150 | --- |

- Application Layer

| Parameter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sender | Receiver | Description | Min [ms] | Max [ms] |
| P2CAN_Client | --- | Timeout for the client to wait after the successful transmission of a request message for the start of incoming response messages. | 50 | N/A |
| P2*CAN_Client | --- | Enhanced timeout for the client to wait after the reception of a negative response message with response code $78 for the start of incoming response messages. | 5000 | N/A |
| --- | P2CAN_Server | Performance requirement for the server to start with the response message after the reception of a request message. | 0 | 20 |
| --- | P2*CAN_Server | Performance requirement for the server to start with the response message after the transmission of a negative response message with response code $78. | 0 | 4900 |
| S3Client | --- | Time between functionally addressed Tester Present $3E request messages transmitted by the client to keep a diagnostic session other than the default session active in multiple servers or maximum time between physical transmitted request messages to a single server. Typical value is 2000ms | - | - |

DIAGNOSTIC MODE AND SERVICE

Diagnostic mode

There are some diagnostic modes (ex. Normal Diagnosis Mode and Extended Diagnostic Mode etc.) for the diagnostic services.

The default mode after IGN ON is Norma Diagnosis Mode. And if the ECU receives the request of Extended Diagnostic Mode from the tester, the ECU enters Extended Diagnostic Mode.

2000msec

| Parameter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sender | Receiver | Description | Min [ms] | Max [ms] |
| --- | S3Server | Time for the server to keep a diagnostic session other than the default session active while not receiving any diagnostic request message. | 5000 | 5100 |

Supported Service

The ECU supports the services in the following table.

| Service Name | SID | Diagnostic Session |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (hex) | Standard Diagnostic Mode |  | Ecu Programming mode |  |
|  |  | Flash ROM normal | C1802 Current failure | Flash ROM normal | C1802 Current failure |
| StartDiagnosticSession | 10 | O | O | (NR$80) | (NR$80) |
| ECUReset | 11 | (NR$80) | (NR$80) | O | O |
| ClearDiagnosticInformation | 14 | O | (NR$11) | (NR$80) | (NR$11) |
| ReadDTCByStatus | 18 | O | O | (NR$80) | (NR$80) |
| ReadEcuIdentification | 1A | O | O | O | O |
| StopDiagnosticSession | 20 | (NR$80) | (NR$80) | O | O |
| ReadDataByLocalIdentifier | 21 | O | (NR$11) | (NR$80) | (NR$11) |
| SecurityAccess | 27 | O | O | O | O |
| InputOutputControlByLocalIdentifier | 30 | O | (NR$11) | (NR$80) | (NR$11) |
| Request Download | 34 | (NR$80) | (NR$80) | O | O |
| Transfer Data | 36 | (NR$11) | (NR$80) | (NR$11) | O |
| Request Transfer Exit | 37 | (NR$11) | (NR$80) | (NR$11) | O |
| WriteDataByLocalIdentifier (RLI$98, $99) | 3B | (NR$80) | (NR$80) | O | O |
| WriteDataByLocalIdentifier (Other supported RLI except $98, $99) | 3B | O | (NR$12) | (NR$80) | (NR$12) |
| WriteDataByLocalIdentifier (not supported RLI) | 3B | (NR$12) | (NR$12) | (NR$12) | (NR$12) |
| TesterPresent | 3E | O | O | O | O |
| Other not supported Service | - | (NR$11) | (NR$11) | (NR$11) | (NR$11) |

(O: Supported)

DIAGNOSTIC SERVICE

Start Diagnostic Session (SID$10)

ECU enters the requested diagnostic session.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $10 | Diagnostic Session Control SID |
| 2 | $85 | ECU Programming Mode |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $50 | Diagnostic Session Control Positive response SID |
| 2 | $85 | ECU Programming Mode |

- Negative response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $10 | Diagnostic Session Control SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | Sub-Function not supported Invalid format | 2 |
| $22 | The condition to start is not satisfied. | 3 |
| $78 | Requested processing cannot be finished within P2Server | 4 |
| $80 | Service not supported inactive Diagnostic Mode | 1 |

- System behavior at execution

Within 2 seconds from IG ON

Within 2 seconds from system reset

Include a self shutdown processing

Include a self shutdown processing

Tester control state

ECU shall enable the diagnostic communication within 2 sec after IG ON. And ECU shall not enable the communication during IG OFF including self-shutdown process.

In addition, the transition shall be done between the Standard Diagnostic Mode and the tester control state by Service $30 as the following.

1 Standard Diagnostic Mode -> Service $30 tester control state

- When ECU receives a specific command by Service $30 and it sends positive response (Only if it is needed to keep the state by S3 handling).

2 Service $30 tester control state -> Standard Diagnostic Mode

- When the condition to end the specified control is satisfied

・When S3Server communication time is not satisfied (S3 timeout)

・When it will be in a communication stop condition due to IG OFF

・AT ECU power down (including system reset)

Clear Diagnostic Information (SID$14)

This service clears the stored DTC and FFD in RAM and EEPROM.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $14 | Clear Diagnostic Information SID |
| 2 | $FF | DTC Group (High byte) |
| 3 | $00 | DTC Group (Low byte) |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $54 | Clear Diagnostic Information Positive response SID |
| 2 | $FF | DTC Group(High Byte) |
| 3 | $00 | DTC Group(Low Byte) |

- Negative response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $14 | Clear Diagnostic Information SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | Message length is not 3 DTC Group is not $FF00 | 2 |
| $22 | The condition to start is not satisfied. | 3 |
| $78 | Requested processing cannot be finished within P2Server | 4 |
| $80 | Service not supported inactive Diagnostic Mode | 1 |

- System behavior at execution

The system behavior will be the following.

| DTC | DTC status | DTC |
| --- | --- | --- |
| stored | Exists only DTC with past status | Positive response is sent after DTC clear |
|  | Exists only DTC with current status | Send NRC$22 (No DTC is cleared) |
|  | Exists DTC with past/current status | Clear only DTC with past status and send NRC$22 after Clear DTC |
| Not stored | - | Send Positive response |

Read DTC By Status (SID$18)

This service outputs the stored DTC

Read DTC by Status Mask

This sub-function outputs the DTC stored in the ECU according to the requested DTC Status Mask.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $04 | Amount of bytes to be evaluated |
| 1 | $18 | Read DTC Information SID |
| 2 | $00 | DTC Status |
| 3 | $FF | DTC Group(High Byte) |
| 4 | $00 | DTC Group(Low Byte) |

- Positive response (DTC is stored)

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $XX | Amount of bytes to be evaluated |
| 1 | $58 | Read DTC Information Positive response SID |
| 2 | $XX | Number of stored DTC |
| 3 | $XX | DTC #1 (High Byte) |
| 4 | $XX | DTC #1 (Low Byte) |
| 5 | $XX | Status #1 |
| : | : | : |
| n-2 | $XX | DTC #m (High Byte) |
| n-1 | $XX | DTC #m (Low Byte) |
| N | $XX | Status #m |

- Positive response (no DTC is stored)

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $58 | Read DTC Information Positive response SID |
| 2 | $00 | Number of stored DTC |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $18 | Read DTC Information SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | Message length is not 4 DTC Status is not $00 DTC Group is not $FF00 | 2 |
| $80 | Service not supported inactive Diagnostic Mode | 1 |

- Structure of status

| Bit | Description |
| --- | --- |
| 7 | Warning Indicator Request State(not supported F06 release) 0 : WL is not turned ON 1 : WL is turned ON |
| 6,5 | 00 : No failure 01 : Past Failure 10 : unused 11 : Present failure |
| 4 | fixed to 0 |
| 3,2,1,0 | 0000 : no DTC 0001 : High voltage(not supported) 0010 : Low voltage(not supported) |

Read ECU ID (SID: $1A)

This service shows ECU ID etc., which is stored in EEPROM, according to the requested sub-function parameter. The ECU reads the data in EEPROM when it receives this request, and it outputs the data.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $1A | Read ECU ID Request SID |
| 2 | $XX | IO |

- Supported IO

| IO | Data name | Data Size [Byte] | ABS | ESP |
| --- | --- | --- | --- | --- |
| $8A | SUZUKI Parts Number | 16 | X | X |
| $8B | ECU Base Hardware Number | 11 | X | X |
| $8D | ECU Software Version Number | 2 | X | X |
| $8E | Component ID | 16 | X | X |
| $98 | Repair shop code or tester serial number | 10 | X | X |
| $99 | Programming data | 4 | X | X |
| $9E | Software ID | 5 |  | X |

X : Supported

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $XX | Amount of bytes to be evaluated |
| 1 | $5A | Read ECU ID Positive Response SID |
| 2 | $XX | IO |
| 3 | $XX | Data#1 |
| : | : | : |
| N | $XX | Data #N |

- Negative response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Read Data By Local Identifier Request SID |
| 2 | $1A | Record Local Identifier |
| 3 | $XX | Negative response code |

- Negative response code

| NRC | Description |
| --- | --- |
| $12 | IO not supported Invalid format |

Read Data By Local Identifier (SID$21)

[01]

This service outputs the current data in the ECU (ex. Wheel speeds and sensor value etc.) according to the requested Data Identifier. The output data is raw value. Then, the ECU doesn’t output the fixed data which specified as the abnormal value even if it has any failure.

About each Data Identifier, see the chapter “RLI stored Data Table”.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $21 | Read Data By Identifier SID |
| 2 | $XX | RLI |

- Supported RLI()

| RLI | Data name | Data Size [Byte] | ABS | ESP MT | ESP AT |
| --- | --- | --- | --- | --- | --- |
| $00 | ABS / ESP-related data | 50 | X | X | X |
| $30 | Software Information of Non Protected Area * Software Version Name | 11 | X | X | X |
| $31 | Software Information of Protected Area * Flashloader Version Name | 11 | X | X | X |
| $50 -$5E* | Stored Data for DTCs | 60 | X | X | X |
| $5F | Stored Data for DTCs (reserved) | - |  |  |  |
| $60 -$63 | Stored Data for AEB Drive Recorder | 50 |  |  |  |
| $64 -$7F | Stored Data for Drive Recorder (reserved) | - |  |  |  |
| $80 -$83 | Stored Data for starting ABS function | 50 | X | X | X |
| $84 -$87 | Stored Data for starting ESP function | 50 |  | X | X |
| $88 -$8B | Stored Data for starting Brake assist function for insufficient booster vacuum during engine stop | 50 |  |  | X |

* In case of ABS project, Stored Data for DTC is $50-$59.

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $XX | Amount of bytes to be evaluated |
| 1 | $21 | Read ECU ID Positive Response SID |
| 2 | $XX | RLI |
| 3 | $XX | Data#1 |
| : | : | : |
| N | $XX | Data #N |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $21 | Read DTC Information SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | RLI not supported Invalid format | 2 |
| $80 | Service not supported inactive Diagnostic Mode | 1 |

- Structure of data

Refer to RID list

Security Access (SID$27)

This service is used to change ECU information protection and ECU controls restriction state (hereafter referred to as ECU lock) into unlock state (hereinafter referred to as ECU unlock).

・By the time this mode is executed, execution of SAE J2186 regulated SEED & KEY procedure, establish restriction on execution of some modes / ECU controls to protect information associated with communication and the vehicle.

Seed Request (AM=$01)

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $27 | Security Access SID |
| 2 | $01 | AM(request Seed) |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $04 | Amount of bytes to be evaluated |
| 1 | $67 | Security Access Positive response SID |
| 2 | $01 | AM |
| 3 | $XX | Security seed #1 |
| 4 | $XX | Security seed #2 |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $27 | Security Access SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | ・AM of request message is not supproted ・Unexpected message length | 1 |
| $37 | DT is still counting when this request is received from tester | 2 |

- Supplementation

When execution of the requested service is possible for ECU, reply positive response with AM and SEED (2 bytes fixed). SEED value (random values) shall be updated for every request from tester. However, in the case of ECU unlock state, all SEED are set to $00. In the case of ECU lock state, don't use $00$00 and $FF $FF for SEED.

・Delay time (DT = 10s) is set up before receiving this request in principle to assure the security. The set timing and the timeout default of DT timer are set as follows.

| Timing parameter | Action | Event | Content |
| --- | --- | --- | --- |
| DT | Initial start | ECU power on | Start DT timer at ECU startup. When SEED request is received after this timer start, reply negative response RC$37. |
|  |  | ECU reset |  |
|  | Sub-sequent stop | DT timeout | When 10s or more passes after DT timer start, SEED request prohibition by this DT timer is cancelled. |
|  | Sub-sequent start | N_USData.con | Restart DT timer at the time of transmission completion of negative response RC $36 (exceedNumberOfAttempts), or the transmission error detection of this negative response. |

Send Key Request (AM=$02)

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $04 | Amount of bytes to be evaluated |
| 1 | $27 | Security Access SID |
| 2 | $02 | AM (send Key) |
| 3 | $XX | Security Key #1 |
| 4 | $XX | Security Key #2 |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $67 | Security Access Positive response SID |
| 2 | $02 | AM (send Key) |
| 3 | $34 | SAS |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $27 | Security Access SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | ・AM of request message is not supproted ・Unexpected message length | 1 |
| $22 | When execution condition is not satisfied | 2 |
| $35 | When Key is not matched | 4 |
| $36 | Exceeded permitted number of KEY mismatch | 3 |

- Supplementation

・ECU shall manage NumberOfFalseAccessAttempts (NFAA) at the time of KEY check.

In case of NFAA>=2 (at the time of two KEY check or more failure), DT is inserted before receiving the next SEED request.

NFAA initializes to 0 only at the time of IGON or security unlock positive response

・A security released (unlock) ECU returns to lock state when any of the following condition is satisfied in each diag session.

(ECU lock recovery condition)

1.Usual failures diagnostic communications mode (Default session)

･When S3Server communication time is not satisfied (S3 timeout)

・When comminication is stopped due to IG OFF

･At ECU power down (including system reset)

2.ECUProgrammingMode (program session)

･When S3Server communication time is not satisfied (S3 timeout)

･When ”StopDiagnosticSession ($20)” is received and replied positive response.

・When comminication is stopped due to IG OFF

・When ”ECUReset($11)” is received and replied positive resposen

・At ECU power down (including system reset)

・Don't use SEED$0000 for KEY operation. For the compare request (sendKey) after security release state (unlock) transition and occurrence of SEED ($0000), judge the entire KEY transmitted by the tester as mismatch, execute management of NFAA and DT, and reply negative response as well. However, unless the ECU lock recovery conditions are satisfied, regardless of the negative response by KEY check result mismatch etc., ECU unlock status is maintained.

Input Output Control By Local Identifier (SID$30)

ECU controls in accordance with parameters Input Output Local Id (IOLI),Input Output Control Parameter (IOCP), the Control State(CS).

Further, in this service, there is a case of using the S3 timer in response to the control content

(See <IOLO Support List>)

Enable / Disable flag is set in ES1. In case of the control object that has Enable status, the control is executed according to tester request and ECU sends positive response.

In case of the control object that has Disable status, normal control is executed (or return to normal control) and ECU sends positive response. (In Disable case, it does not matter regarding value of the bits that support the CS2 ~ 4, but if the bits of the non-support is used, RC $ 12 negative response is sent.) If ECU receives a request the control that does not use S3 timer and this has Disable state, ECU send positive response with normal control state.

When the control is executed with S3 timer, system reset is executed by S3 timeout. If S3 timer is not used, system reset is executed after the specified control finished.

If ECU send negative response except for NR$78 during S3 timer control execution, ECU stops the control but not stop S3 timer. So system reset is executed after S3 timeout.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $XX | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $XX | IOLI |
| 3 | $XX | IOCP |
| 4 | $XX | CS1* |
| 5 | $XX | CS2* |
| 6 | $XX | CS3* |
| 7 | $XX | CS4* |

(*) CS1-4 are depended on IOCP.

- Supported IOLI

| IOLI | IOCP | System | CS1 |  | CS2 |  | CS3 | CS4 | S3 handling | Reset*1 “1 | ABS | ESP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| $00 | $07 | Valve drive | Bit7 | Air bleeding (secondary) | Bit7 | Air bleeding (secondary) | $00 | $00 | X | X | X | X |
|  |  |  | Bit6 | Calibration(All) | Bit6 | Air bleeding (thirdry) | $00 | $00 | X | X | - | X |
|  |  |  | Bit3 | Pressure decreasing drive | Bit3 | FR Wheel | $00 | $00 | X | X | X | X |
|  |  |  |  |  | Bit2 | FL Wheel | $00 | $00 | X | X | X | X |
|  |  |  |  |  | Bit1 | RR Wheel | $00 | $00 | X | X | X | X |
|  |  |  |  |  | Bit0 | RL Wheel | $00 | $00 | X | X | X | X |
|  |  |  | Bit2 | Pressure-increasing drive | Bit3 | FR Wheel | $00 | $00 | X | X | - | X |
|  |  |  |  |  | Bit2 | FL Wheel | $00 | $00 | X | X | - | X |
|  |  |  |  |  | Bit1 | RR Wheel | $00 | $00 | X | X | - | X |
|  |  |  |  |  | Bit0 | RL Wheel | $00 | $00 | X | X | - | X |
| $01 | $08 | Sensor calibration | Bit7 | Calibration(All) | Bit7 | Calibration | $00 | $00 | - | X | - | X |
| $02 | $07 | Function settings | Bit7 | Pressure-increasing drive | Bit7 | ESP Outage /start-up | $00 | $00 | - | X | - | X |
| $03 | $07 | Various drive | Bit7 | Simple reset ECU | Bit7 | $00 | $00 | $00 | - | X | X | X |
| $80 | $07 | Fill and bleed | Bit7 | Fill and bleed | Bit7 | Filling | $00 | $00 | X | X | X | X |
| $81 | $07 | Security | Bit7 | PASSWORD verification | Bit7 | $00 | $00 | $00 | X | X | - | X |
| $XX | $00 | Return control to ECU | No CS1 ~ CS4 setting |  |  |  |  |  | - | - | - | - |

X : supported

(General)

Above table for each command, send RC $ 12 negative response for non-support.

$XX : IOLI $ 00, $ 02, $ 03, $ 04, $ 80, $ 81 is applicable. ECU returns RC $12 negative response if the IOLI that is not IOLI $ 00, $ 02, $ 03, $ 04, $ 80, $ 81 is received. If this command is received during S3 timer control execution, S3 timer is restarted.

If ECU receives the correct command of IOCP ($00), ECU stop the control execution regardless previous IOLI command and send positive response.

*1 : System reset is simple reset if there is no remark in each IOLI supplementary explanation.

In this case, simple reset shall be executed after S3 timeout if ECU executes S3 timer control. After simple reset, S3 timer is not restarted.

- Supported IOCP

| IOCP | Parameter name | CS | Description |
| --- | --- | --- | --- |
| $00 | Return Control To ECU | N/A | Return to ECU controlled state from tester controlled state |
| $07 | Short Term Coordination | 4byte | ControlOptionParameter' setting to RAM |
| $08 | Long Term Coordination | 4byte | ControlOptionParameter' setting to EEPROM/FLASH ROM etc. |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $70 | Routine Control SID |
| 2 | $XX | IOLI |
| 3 | $XX | IOCP |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $30 | Routine Control SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $12 | ・When IOLI of request message is not supported ・When IOCP of request message is not supported (When IOCP other than $00 and $07 is set by IOLI that uses S3 timer and when IOCP other than $08 is set by IOLI that do not use S3 timer) ・When unsupported bit is used by CS request message | 2 |
| $22 | Condition to execute not fulfilled | 4 |
| $31 | ・When 2 or more Enable bit is set in case where only single control bit execution is allowed in setting function using Enable/Disable bits (However, this excludes the case where 2 or more driving functions are permitted) ・In case of control execution by bit identifier, when the instruction information of the identifier is not defined ・In case of control execution by numerical value identifier, when the control identifier value is out of range | 3 |
| $78 | Requested processing cannot be finished within P2Server | 5 |
| $80 | Service not supported inactive Diagnostic Mode | 1 |

- Supplementation

・ECU can normally process other service requests ('ReadDataByLocalIdentifier($21) service' etc.), even if it is under the specified control.

・Establish entry conditions and operating constant ranges (limit) in ECU for safety reasons.

・When at least one of the conditions below is satisfied, ECU ends specified control currently in operation, and return to regular ECU control satate.

1. When specified control exceeds the operating range.

2. When S3Server communication time is not satisfied (S3 timeout)

(However, S3 handling may not be applied in some driving functions. The support information on S3 handling is descrbied in 'SUZUKI SDL on CAN-KWP2000 SPECIFICATION PART2.'

3. When other specified control request is received from tester (however, continue such controls when same controls request as is transmitted)

4. When IOCP=$00 ('returnControlToECU') is transmitted from Tester

5. When it changes into communications stopped state by IGOFF (including the state during self shut process)

6. At ECU power down (including System Reset)

7. When 'StartDiagnosticSession($10) DCM:$85 (ECUProgrammingMode)' is received and replied positive response.

Request Valve drive$00 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $00 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | Air bleeding(secondary) 1= Enable 0=Disable |
|  | Bit6 | Air bleeding(thirdly) 1= Enable 0=Disable |
|  | Bit5 | Not define(fix to 0) |
|  | Bit4 | Not define(fix to 0) |
|  | Bit3 | Pressure decreasing drive 1= Enable 0=Disable*1 |
|  | Bit2 | Pressure-increasing drive 1= Enable 0=Disable*2 |
|  | Bit1 | Not define(fix to 0) |
|  | Bit0 | Not define(fix to 0) |
| 5 | Bit7 | Air bleeding(secondary) 1= On 0=Off |
|  | Bit6 | Air bleeding(thirdly) 1= On 0=Off |
|  | Bit5 | Not define(fix to 0) |
|  | Bit4 | Not define(fix to 0) |
|  | Bit3 | FR Wheel 1= On 0=Off |
|  | Bit2 | FL Wheel 1= On 0=Off |
|  | Bit1 | RR Wheel 1= On 0=Off |
|  | Bit0 | RL Wheel 1= On 0=Off |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

(General)

Multiple simultaneous specification by CS1 Bit7, Bit6, Bit3, Bit2 is not allowed.

In this case, ECU send RC $ 31 negative response.

*1 : In case of pressure decreasing drive, it is acceptable to conduct simultaneously driven

by a combination of either CS2 Bit0-Bit3 one.

*2 : In the pressure-increasing drive, it is acceptable to conduct simultaneously driven

by a combination of either CS2 Bit0-Bit3 one.

Air bleeding drive (secondary system)

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), Brake SW ON

2), All wheel speed < 10km/h

3), After the initial check (IG ON later, after one second or more)

4), No failure of valve and pump

5), 9.8V < Battery Voltage < 16.9V

- Drive Pattern

All outlet valve

Motor driven

Brake signal

Valve control state flag at air bleeding time

(Secondary system)

Stop condition satisfied flag at air bleeding time.

(Secondary system)

0 clear neutral condition is satisfied

1 set neutral condition is satisfied (with clear IG OFF)

HU time control (5s)

HU time control (5s)

Command is received

detect

Not detect

- Supplement

・The "stop condition" is outside of "entry and continue condition".

・If a maximum wheel speed (V max) ≧ 10km / h is detected during the operation, ECU execute system reset (simple reset).

・If the stop condition is satisfied during air bleeding and ECU stop it, ECU shall clear Valve control state flag at air bleeding time (set to zero) and set Stop condition satisfied flag at air bleeding time (set to one).

Air bleeding drive (thirdly system)

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), All wheel speed < 10km/h

2), After the initial check (IG ON later, after one second or more)

3), No failure of valve and pump

4), 9.8V < Battery Voltage < 16.9V

- Drive Pattern

Brake signal

Valve control state flag at air bleeding time

(Thirdry system)

Stop condition satisfied flag at air bleeding time (Thirdry system)

Each valve drive

Motor driven

Thirdly system air bleeding pattern

HU time control (31s)

1 set neutral condition is satisfied (with clear IG OFF)

0 clear neutral condition is satisfied

Command is received

detect

Not detect

- Supplement

・The "stop condition" is outside of "entry and continue condition".

・If a maximum wheel speed (V max) ≧ 10km / h is detected during the operation, ECU execute system reset (simple reset).

・If the stop condition is satisfied during air bleeding and ECU stop it, ECU shall clear Valve control state flag at air bleeding time (set to zero) and set Stop condition satisfied flag at air bleeding time (set to one).

Pressure decreasing drive

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), All wheel speed < 10km/h

2), After the initial check (IG ON later, after one second or more)

3), No failure of valve and pump

4), 9.8V < Battery Voltage < 16.9V

- Drive Pattern

Valve

(Inlet and outlet)

Motor

Command is received

- Supplement

・While driving at 10km / h or less, pressure decreasing drive is possible.

・If it is repeated for a long time continuously, it is important to note valve damage in heat generation.

・If pressure decreasing drive command for another wheel is received during pressure decreasing drive, the current control shall be stopped and start pressure decreasing drive according to new request.

Pressure increasing drive

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), After the initial check (IG ON later, after one second or more)

2), Brake SW OFF

3), All wheel speed < 10km/h

4), No failure of valve and pump

5), 9.8V < Battery Voltage < 16.9V

- Drive Pattern

Brake signal

TCS1 (TCS2) Valve

ESV1 (ESV2) Valve

Valve at Pressure increased wheel (Inlet)

Valve at Pressure increased wheel (Outlet)

Other wheel valve

(Inlet)

Other wheel valve

(Outlet)

Motor

Command is received

- Supplement

・If it is repeated for a long time continuously, it is important to note valve damage in heat generation.

・If pressure increasing drive command for another wheel is received during pressure increasing drive, the current control shall be stopped and start pressure increasing drive according to new request.

Request Sensor calibration $01 $08

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $01 | IOLI |
| 3 | $08 | IOCP |
| 4 | Bit7 | Sensor Calibration All 1= Enable 0=Disable |
|  | Bit6-0 | Not define(fix to 0) |
| 5 | Bit7 | Sensor Calibration 1= Execute |
|  | Bit6-0 | Not define(fix to 0) |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

Sensor calibration

- Entry and continue conditions

Common for All sensors

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), After the initial check (IG ON later, after one second or more)

2), All wheel speed <1km / h

3), Engine RPM = 0[rpm] (“Engine running status = 0” – CAN data from ECM)

4), There is no fault in the target sensors for calibration

5), Service $21 RLI$00 Pos48 bit4 “SAS authentication system type” = 1(ESP)

6), Not “Variant coding not finished” (C1082 not detected)

Steering angle sensor

・Steering angle sensor≦±15°(*2

Master cylinder pressure sensor

1) Pressure sensor≦±1.61Mpa

2) Brake SW OFF

Longitudinal G sensor

・G sensor signal ≦ 0.25G

Lateral G sensor

・G sensor signal ≦ 0.25G (*3

*: Use RC$78 negative response for this command.

*1: The condition of command execution is only entry conditions.

*2: “within ± 5 °” is the recommended value (However, this can be a indication of vehicle standstill on flat

Because sensor values (Mode$21 LID$08 Pos # # 25,# 26) during calibration not finished are

"$ 0000 (fixed)" output)

*3: within ± 0.05G is recommended value.

- Supplement

•Calibration targets are 4 sensors: steering angle sensor / master cylinder pressure sensor / G sensor (Long / Lat).

•When ECU receives this command, ECU execute calibration for all the relevant sensor. After the sensor calibration started, ECU continues all function disable (all WL on) until IGN OFF or system reset.

• If the access to the EEPROM by the sensor calibration (writing) is successful, ECU sends positive response after clear of DTC of sensor not calibration (C1074, C1075, C1076, C1078) and system reset immediately (simple reset).

• If the processing cannot be completed by some reasons (e.g. EEPROM access failure) even command is ok and specified control can be executed, specified control should be stopped and return RC$10 negative response according to the sequence of positive response above.

• If ECU cannot confirm the completion of the calibration of steering angle sensor after 3 sec past from calibration start request, ECU calibration information is registered in EEPROM (calibration is completed from ESP point of view) and send RC$10 negative response and system reset. In this case, the status is the calibration of steering angle sensor not completed yet and DTC C1024 and C1075 are is registered.

-ECU behavior during the sensor calibration is as follows.

<STEP1>

If ECU check only No. 5 of entry conditions and SAS recognized system type is not “1 (ESP)”, ECU finish this treatment and returns the RC $ 22 negative response.

For regular calibration request from the tester, erases the calibration information of the sensor, and sets DTC of the calibration not finished.

<STEP2>

ECU check all of the entry conditions for corresponding all sensors.

If at least one of the above entry conditions is not fulfilled, ECU returns RC $ 22 negative response and system reset.

• If all entry conditions are satisfied, go to STEP3.

<STEP3>

• After calibration information register to the EEPROM, ECU deletes all DTC of the sensor calibration not finished and reset the system (simple reset).

Request Function settings $02 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $02 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | Not define(fix to 0) |
|  | Bit6 | ESP function setting 1= Enable 0=Disable |
|  | Bit5-0 | Not define(fix to 0) |
| 5 | Bit7 | Not define(fix to 0) |
|  | Bit6 | ESP function 1= Stop 0=Start |
|  | Bit5-0 | Not define(fix to 0) |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

ESP function disable - vehicle inspection mode

- Entry and continue conditions

• After initial check (IG ON later, after one second or more)

*Even in initial check, if the CORE is acceptable, ECU send positive response.

• All wheel speed < 15km/h. If this condition is not fulfilled, ECU should reply NRC $22.

- Supplement

• This command is supported only ESP

•In case of disable request, system reset is not simple reset. In case of enable request, system reset is simple reset.

• If you have received an ESP function disable command, ECU move to ESP function disable state (ESP W / L, ESP ACT / L, ESP OFF / L ON), and ECU sends back positive response.

• After starting ESP function disable mode, ESP function disable is continued until ESP enable command receive or IGN OFF. If ECU receives the Disable specification request, ECU continues ESP function disable mode and positive response. ESP function disable mode cannot be finished by ESP OFF SW operation, vehicle speed conditions and system reset. And ESP keep normal failure detection condition and store DTC.

•Even if ECU is in ESP function disable mode, pressure decreasing/increasing drive are allowed.

• If ECU have received an ESP function enable command, ECU send positive feedback after control execution finished and perform system reset (simple reset) once. After that, ECU cancel ESP function disabled mode after initial check finished. ( ESP W / L, ESP ACT / L, ESP OFF / L OFF).

Request Various drive $03 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $03 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | ECU simple reset 1= Enable 0=Disable |
|  | Bit6-0 | Not define(fix to 0) |
| 5 | $00 | Not define |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

ECU force reset

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), After initial check (IG ON later, after one second or more)

2), During S3 timer driven by Service $ 30

- Supplement

・W hen ECU receives this command, ECU send positive response and after that, ECU to perform a simple reset. In this case, S3 timer is not restarted.

・If ECU cannot perform reset by some reason, ECU sends back the RC $ 10 negative response, and restart S3 timer.

Request Various drive 2 $04 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $04 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | Brake Lamp 1= Enable 0=Disable |
|  | Bit6-0 | Not define(fix to 0) |
| 5 | Bit7 | Brake Lamp 1= On 0=Off |
|  | Bit6-0 | Not define(fix to 0) |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

Brake Lamp

- Entry and continue conditions

apply the following entry conditions. If any condition is not fulfilled, RC $ 22 negative response.

(If ECU cannot judge Brake SW, wheel speed, the entry condition is treated as “Satisfied”)

1), Brake SW = OFF

2), All wheel speed < 10km/h

- Supplement

• If ECU receives this command, ECU send positive response at the brake lamp driving.

• The brake lamp driving continues for 3 seconds after request receiving.

• If entry and continue condition is not fulfilled during operation, stop the brake lamp drive.

Request Fill and bleed $80 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $80 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | Fill and Bleed 1= Enable 0=Disable |
|  | Bit6-0 | Not define(fix to 0) |
| 5 | Bit7 | Filling 1= On 0=Off |
|  | Bit6-0 | Not define(fix to 0) |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

Fill and bleed

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1), All wheel speed ≦ 2km/h

2), Fill and bleed completion flag = 0

3), After initial check (IG ON later, after one second or more)

4), No failure of valve, pump

5), 9.8V < Battery Voltage < 16.9V

- Drive Pattern

Detect

30 80 07 80 00

or

30 80 07 00 00

or

30 80 00

Filli and bleed completion flag

Filling execution flag

Control state flag at Fill and bleed driving

Fluid filling flag

ESV Valve (*1

Outlet valve

(FL/FR)

Outlet valve

(RL/RR)

Motor

Not detect

detect

3B 80 11

30 80 07 80 80

A : Fill and bleed start command (Service $30 ILOI $80)

When drive start command is received (at A in the figure), ECU executes the following process

1) all pressure reducing valve ON

2) Fill and bleed execution flag set to 1

3) Control state flag set to 10

4) DTC C107F set as current state

B : Brake fluid filling flag write command (Service $3B RLI $80)

When Brake fluid filling flag write command is received (At B in the figure), ECU executes the following process

1), Fluid fill flag set to 1 (Flag is stored in EEPROM)

2), start motor driving

• This command can be executed only 20ms or later after command A running (positive response). If this command is received within 20ms from command A running, ECU returns RC $ 22 a negative response

C : Fill and bleed drive stop command (Service $30 ILOI $80)

When Fill and bleed drive stop command is received (At C in the figure), ECU executes the following process. If this process cannot be completed within P2CAN_SERVER, ECU returns RC$78 negative at first, and then, send positive response when the following to (1), (2) (3) processes are completed.

1), All pressure reducing valves are OFF

2), Clear Control state flag (set to 0)

3) Motor is ON for 2 sec to accumulate brake fluid from LPA

4), ESV valve ON for 1 second (For adjustment of brake fluid)

• This command can be executed only 5s+20ms or later after command B running (positive response).If receive this command within this time, ECU replies positive response.

ECU should execute above process - C, if any of the following conditions are fulfilled:

1: ECU receives IOLI$80 CS1 Bit7 Disable command(correctly format command)

2: ECU receives IOLI$80 CS2 Off command(correctly format command)

3: ECU receives IOLI$80 ReturnControlToECU command (correctly format command)

C’ : After liquid discharge drive

Clear DTC C107F

D : Fill and bleed completion flag write command (Service $3B RLI $80)

When Fill and bleed completion flag write command is received (At D in the figure), ECU executes the following process. If this process cannot be completed within P2CAN_SERVER, ECU returns RC$78 negative at first, and then, send positive response when the following to all processes are completed.

1), Fill and bleed completion flag set to 1 (Flag is stored in the EEPROM)

2), DTC C1081 is cleared

- Supplement

- If wheel speed> 2km / h during Fill and bleed driving, system reset (simple reset) is executed.

- When stop conditions are satisfied and it stops during Fill and bleed driving, Valve control state flag is cleared (set to 0)

- If Fill and bleed completion flag is 0, system state will become the following 1) to 3).

1), ABS WL is flashing (On/OFF) with intervals of 0.5 seconds, Other all WLs are ON. However, ABS WL is ON if any failure related to fill and bleed is detected.

2),The threshold of low voltage detection down from 10V to 7V

3), All control functions are disabled.

Request Security $81 $07

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $07 | Amount of bytes to be evaluated |
| 1 | $30 | Routine Control SID |
| 2 | $81 | IOLI |
| 3 | $07 | IOCP |
| 4 | Bit7 | Password Verification 1= Enable 0=Disable |
|  | Bit6-0 | Not define(fix to 0) |
| 5 | $00 | Not define |
| 6 | $00 | Not define |
| 7 | $00 | Not define |

Password verification

- Entry and continue conditions

Apply the following entry conditions. If any condition is not fulfilled, RC $ 22 negative response.

1), After initial check (IG ON later, after one second or more)

*Even in initial check, if the CORE is acceptable, ECU send positive response.

2), No ECU internal failure (C1071 as current failure)

3), All wheel speed <10km / h

- Supplement

•Only ESP support this command

•If the ECU has transitioned to “Unlocked mode (variant data can be registered)” by password verification, this state shall be kept until the specified control termination conditions that are described in "SUZUKI SDL on CAN-KWP2000 SPECICATION PART1 for CHASSIS & BODY" Service $ 30 are fulfilled.

•If Password verification is not satisfied, ECU sends NRC $ 31 negative response.

•When ECU receives the request of password verification in ECU unlocked mode, ECU keeps unlock mode and sends positive response. However, if ECU receives wrong password verification request, ECU sends RC$31 negative response and return to ECU lock mode.

•When ECU receives any command from tester except for the case that ECU will return NR$78 in ECU unlock mode, ECU sends RC$31 negative response and ECU shall returns to ECU lock mode.

• Regarding password information, follow the instructions of SUZUKI chassis design department.

Write Data By Local Identifier (SID$3B)

This service requests the ECU to write the request data into EEPROM according to Data Identifier.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $3B | Write Data By Identifier SID |
| 2 | $XX | RLI |
| 3 | $XX | RV1 |

- Supported RLI

| RLI | Description | Data Size | ABS | ESP |
| --- | --- | --- | --- | --- |
| $80 | Fill and bleed related flags writing function | 1byte | X | X |
| $81 | ABS / ESP variant registration related function | 1byte |  | X |
| $98 | Repair shop code or tester serial number | 10byte | X | X |
| $99 | Programming date | 4byte | X | X |

- Positive response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $02 | Amount of bytes to be evaluated |
| 1 | $7B | Write Data By Identifier Positive response SID |
| 2 | $XX | RLI |

- Negative Response

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $03 | Amount of bytes to be evaluated |
| 1 | $7F | Negative response |
| 2 | $3B | Write Data By Identifier SID |
| 3 | $XX | Negative response code |

- Negative response code (NRC)

| NRC | Description | Priority |
| --- | --- | --- |
| $10 | ・General Reject | 6 |
| $12 | ・RLI of request message is not supported. ・The data length to write in differs from the one expected ・Unexpected message length | 1 |
| $22 | ・When deviated from control execution condition at the time of reception of request message. | 4 |
| $31 | ・When requested to write in invalid data | 3 |
| $33 | ･When ECU is in lock state (When the RLI makes security lock cancelation as the execution conditions) | 3 |
| $78 | ・When positive response or negative response (except RC$78) cannot be returned within P2 CAN_Server_Max or P2*CAN_Server_Max time during ECU internal processing, | 5 |
| $80 | ･The case where this RLI execution is not in permitted ECU mode | 2 |

Fell and bleed related flags writing RLI$80

| RLI | Bit | Description | ABS | ESP |
| --- | --- | --- | --- | --- |
| $80 | 0 | Fill and bleed execution flag 1= Act 0=Not Act | X | X |
|  | 1 | Fill and bleed completion flag 1= Complete 0=Not Complete | X | X |
|  | 2 | Reserved |  |  |
|  | 3 | Reserved |  |  |
|  | 4 | Fluid filling flag 1= Filling 0=Stop | X | X |
|  | 5 | Reserved |  |  |
|  | 6 | Reserved |  |  |
|  | 7 | Reserved |  |  |

X : Supported

All Fill and bleed related flags that are defined table above set to “1” by pressure decreasing /increasing drive by Service $ 30 ILOI $ 00

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

<Fill and bleed drive by SID $ 30 ILOI $ 80: Current status is “not activated” >

No conditions

< Fill and bleed drive by SID $ 30 ILOI $ 80: Current status is “activating” >

20ms or more after Fill and bleed drive start

- Supplement

If the command for writing of “Reserved” bit is received as "1", ESC send RC$31 negative response.

When ECU receives $00 writing command, ECU should clear DTC C107F and all flags.

During detecting C107F or if ECU has never executed Air bleeding drive, ECU reply RC22 when ECU received $13 write request command.

Experience of Air bleeding

If ECU has executed Fill and Bleed or Air Bleeding drive, Experience of Air Bleeding should be done.

variant registration RLI$81

- Entry and continue conditions

Apply all the entry conditions below. If any condition is not fulfilled, RC $ 22 negative response.

1),Service $ 30 ILOI $ 81 password verification is established successfully.

2), No the following current failure

-C1020 master cylinder pressure sensor system power failure

-C1041 holding valve solenoid failure (FR)

-C1042 pressure reducing valve solenoid failure (FR)

-C1043 switching solenoid valve defect (solenoid 1)

-C1044 switching solenoid valve defect (solenoid 2)

-C1045 holding valve solenoid failure (FL)

-C1046 pressure reducing valve solenoid failure (FL)

-C1051 holding valve solenoid failure (RR)

-C1052 pressure reducing valve solenoid failure (RR)

-C1053 accumulator solenoid valve defect (solenoid 1)

-C1054 accumulator solenoid valve defect (solenoid 2)

-C1055 holding valve solenoid failure (RL)

-C1056 pressure reducing valve solenoid failure (RL)

-C1057 power supply voltage abnormality

-C1061 pump motor relay failure

-C1063 fail-safe relay failure

-C1071 ECU internal fault

3),Variant data is not finished (C1082 is registered) state (* 1

- Supplement

• When ECU receives this command, ECU executes variant registration and send positive response, after that, system reset is executed.

• If the variant data that is out of definition required for "Base Hardware Parts Number", ECU send RC $ 31 negative response. However, if variant data is default value ($FF), ECU executes default variant registration and send positive response. After that, system reset is executed.

• For variant data registration details, refer to the specifications that we chassis design department issued.

RLI$98, 99

･For EEPROM writing data integrity, store the checksum of written RLI in the EEPROM. If this service is proper, the newly written RLI reflected checksum value, together with requested RLI data from tester, is written.

･When tester is not able to receive the response of ECU in a normal way, the same request as last time may be resent. In this case, it processes as a new request

< The table of RLI $98 Repair shop code or tester serial number >

| Position | Parameter name | Min | Max |
| --- | --- | --- | --- |
| 1 | RSCOTSN #1 | - | - |
| 2 | RSCOTSN #2 | - | - |
| 3 | RSCOTSN #3 | - | - |
| 4 | RSCOTSN #4 | - | - |
| 5 | RSCOTSN #5 | - | - |
| 6 | RSCOTSN #6 | - | - |
| 7 | RSCOTSN #7 | - | - |
| 8 | RSCOTSN #8 | - | - |
| 9 | RSCOTSN #9 | - | - |
| 10 | RSCOTSN #10 | - | - |

<The table of RLI $99 Programming date >

| Position | Parameter name | Min | Max |
| --- | --- | --- | --- |
| 1 | PD #1 | - | - |
| 2 | PD #2 | - | - |
| 3 | PD #3 | - | - |
| 4 | PD #4 | - | - |

These Values can be read by SID$1A.

| Execution permission conditions (RLI$98,$99) | The following conditions are applied. If any condition is not satisfied, send negative response according to the negative response list written in Part1 specifications (Service $3B). 1) It is ECUProgrammingMode 2) Security is in released state by Service$27 |
| --- | --- |
| Supplemental remarks | ･To return to initial state, write dummy value (All $FF) using appropriate RLI. ･When normal command is received, ECU sends positive response after successful completion of processing. ・Although command is normal and assigned control is possible to execute, if it is not possible to complete processing for some reasons (communication error, EEPROM access failure, etc.), send negative response $10. |

Tester present (SID$3E)

・This service is used in order to maintain DIAG session other than the default session(for example, program sessions etc.). Moreover, it

is used in the cases (for example, actives control states by 'InputOutpurtControlByLocalIdentifier($30) service' etc.) where S3 handling is

needed besides the above application.

・Tester transmits request message without parameter

・When requested service is possible for execution, ECU replies positive message without parameter.

- Request message

| Byte | Data | Contents |
| --- | --- | --- |
| ML | $01 | Amount of bytes to be evaluated |
| 1 | $3E | Tester Present SID |

- Positive Response

| ML | $01 | Amount of bytes to be evaluated |
| --- | --- | --- |
| 1 | $7E | Tester Present SID |

- Negative Response

| ML | $03 | Amount of bytes to be evaluated |
| --- | --- | --- |
| 1 | $7F | Negative response |
| 2 | $3E | Tester Present SID |
| 3 | $12 | Invalid format |

RLI stored Data Table

[01]

RLI $00

| Position | bit | Parameter Name | min | Max | Factor | Offset | Unit | ABS |  |  | ESP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | MT |  | A T | M T | AT |  |
|  |  |  |  |  |  |  |  | With IS | With Out IS |  |  | With IS | With out IS |
| 1 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | RF Wheel speed sensor | 0 | 255 | 1 | - | Km/h | X | X | X | X | X | X |
| 9 |  | LF Wheel speed sensor | 0 | 255 | 1 | - | Km/h | X | X | X | X | X | X |
| 10 |  | RR Wheel speed sensor | 0 | 255 | 1 | - | Km/h | X | X | X | X | X | X |
| 11 |  | LR Wheel speed sensor | 0 | 255 | 1 | - | Km/h | X | X | X | X | X | X |
| 12 |  | G sensor signal (*1 (*4 | -1.7 (-2.56) | 1.7 (2.54) | 0.02 | -2.56 | G |  |  |  | X | X | X |
| 13 |  | Battery voltage | 0 | 18.1 (20) | 20/255 | - | V | X | X | X | X | X | X |
| 14 |  | Motor relay monitor voltage | 0 | 14.8 (20.5) | 20.5/255 | - | V | X | X | X | X | X | X |
| 15 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  | Vacuum sensor*17 | 0 | 127.5 | 0.5 | - | KPa |  |  |  |  | X | X |
| 18 |  | Master cylinder pressure sensor (*4 | -2 | 23.5 | 0.1 | -2 | MPa | X | X | X | X | X | X |
| 19 |  | Lateral G sensor signal (*2 (*4 | -1.7 (-2.56) | 1.7 (2.54) | 0.02 | -2.56 | G |  |  |  | X | X | X |
| 20 |  | Yaw rate sensor(High byte) (*2 | -75 (-1638.4) | 75 (1638.35) | 0.05 | -1638.4 | deg/s |  |  |  | X | X | X |
| 21 |  | Yaw rate sensor(Low byte) (*2 |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  | Engine RPM (High byte) | 0 | 16383.75 | 0.25 | - | rpm | X | X | X | X | X | X |
| 24 |  | Engine RPM (Low byte) |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | Steering angle(High byte) (*2 (*3 (*5 | -850 (-32768) | 850 (32767) | 1 | - | deg |  |  |  | X | X | X |
| 26 |  | Steering angle(Low byte) (*2 (*3 (*5 |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  | ABS/ESP Variant Data |  |  |  |  |  |  |  |  | X | X | X |
| 32 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 39 | Bit0 | Variant Coding Mismatch Detection status (*18 | $0 :Not completed $1: Completed |  | - | - | - |  |  |  | X | X | X |
|  | Bit1 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit2 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit3 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | MMW(*16 | $0 :Not Equipped $1: Equipped |  | - | - | - | X | X | X | X | X | X |
|  | Bit6 | MFL(*16 | $0 :Not Equipped $1: Equipped |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 | Pos 39 support information(*15 | $0:support $1:not support |  | - | - | - | X | X | X | X | X | X |
| 40 | Bit0 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit1 | HSA (*13 | $0:not In control $1:In control |  | - | - | - |  |  |  |  | X | X |
|  | Bit2 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit3 | Brake lamp relay (monitor value) | $0:Lamp Off $1:Lamp On |  | - | - | - |  |  |  |  |  |  |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit6 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit7 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
| 41 | Bit0 | Brake SW | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit1 | Fluid-filled flag | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit2 | Fill and bleed execution flag | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit3 | Fill and bleed completion flag | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit4 | (Reserved) | Fixed to 0 |  |  |  |  |  |  |  |  |  |  |
|  | Bit5 | Valve control state flag at fill and bleed | $0:not In control $1:In control |  | - | - | - | X | X | X | X | X | X |
|  | Bit6 | Valve control state flag at air bleeding (Secondary system) | $0:not In control $1:In control |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 | Stop condition of air bleeding is satisfied flag (Secondary of system) | $0:False $1:True |  | - | - | - | X | X | X | X | X | X |
| 42 | Bit0 | ABS control | $0:not In control $1:In control |  | - | - | - | X | X | X | X | X | X |
|  | Bit1 | ESP control | $0:not In control $1:In control |  | - | - | - |  |  |  | X | X | X |
|  | Bit2 | TCS control (Brake) | $0:not In control $1:In control |  | - | - | - |  |  |  | X | X | X |
|  | Bit3 | TCS control (Engine) | $0:not In control $1:In control |  | - | - | - |  |  |  | X | X | X |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit6 | Valve control state flag air bleeding at the time (3-order system) | $0:not In control $1:In control |  | - | - | - |  |  |  | X | X | X |
|  | Bit7 | Stop condition is satisfied flag air bleeding during (3-order system) | $0:False $1:True |  | - | - | - |  |  |  | X | X | X |
| 43 | Bit0 | ABS&EBD&ESP OFF State (*6 | $0:ABS&EBD& ESP_ON $1:ABS&EBD& ESP_OFF |  | - | - | - | X | X | X | X | X | X |
|  | Bit1 | ESP OFF Status Normal mode-control value (*7 | $0:ESP_ON $1:ESP_OFF |  | - | - | - |  |  |  | X | X | X |
|  | Bit2 | ESP OFF Status Vehicle inspection mode - control value (*8 | $0:ESP_ON $1:ESP_OFF |  | - | - | - |  |  |  | X | X | X |
|  | Bit3 | ESP OFF Status(Monitor value) | $0:Off $1:On |  | - | - | - |  |  |  | X | X | X |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit6 | Steering angle sensor signal (*9 | $0:Neutral $1:Not Neutral |  | - | - | - |  |  |  | X | X | X |
|  | Bit7 | Vacuum SW(*14 | $0:Off $1:On |  | - | - | - | X |  | X |  |  |  |
| 44 | Bit0 | Holding valve solenoid state (FR) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit1 | Pressure reducing valve solenoid state (FR) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit2 | Holding valve solenoid state (FL) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit3 | Pressure reducing valve solenoid state (FL) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit4 | Holding valve solenoid state (RR) Monitor | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit5 | Pressure reducing valve solenoid state (RR) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit6 | Holding valve solenoid state (RL) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 | Pressure reducing valve solenoid state (RL) Monitor value | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
| 45 | Bit0 | Switching solenoid valve(MCI) state (FR / RL system) monitor value | $0:Off $1:On |  | - | - | - |  |  |  | X | X | X |
|  | Bit1 | Switching solenoid valve (MCI) state (FL / RR system) monitor value | $0:Off $1:On |  | - | - | - |  |  |  | X | X | X |
|  | Bit2 | Accumulator solenoid valve (LPF) state (FR / RL system) monitor value | $0:Off $1:On |  | - | - | - |  |  |  | X | X | X |
|  | Bit3 | Accumulator solenoid valve(LPF) state (FL / RR system) monitor value | $0:Off $1:On |  | - | - | - |  |  |  | X | X | X |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit6 | Pump motor relay (monitor value) | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 | Valve relay (monitor value) (*10 | $0:Off $1:On |  | - | - | - | X | X | X | X | X | X |
| 46 | Bit0 | System Type | $0:ABS $1:ESP |  | - | - | - | X | X | X | X | X | X |
|  | Bit1 | Drive Wheel Type | $0 : 2WD $1 : 4WD/4HF $2-$3 : (Reserved) |  | - | - | - | X | X | X | X | X | X |
|  | Bit2 |  |  |  | - | - | - | X | X | X | X | X | X |
|  | Bit3 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit4 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit5 | Transmission Type | $0 : MT $1 : AT $2 : CVT $3 : AMT $4-$7 : (Reserved) |  | - | - | - | X | X | X | X | X | X |
|  | Bit6 |  |  |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 |  |  |  | - | - | - | X | X | X | X | X | X |
| 47 |  | (Reserved) |  |  | - | - | - |  |  |  |  |  |  |
| 48 | Bit0 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit1 | HSA | $0 :Not Equipped $1: Equipped |  | - | - | - | X | X | X | X | X | X |
|  | Bit2 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
|  | Bit3 | Idle stop specification | $0 :Not Equipped $1: Equipped |  | - | - | - | X | X | X | X | X | X |
|  | Bit4 | SAS authentication system type (* 11 | $0:Not ESP $1:ESP |  | - | - | - |  |  |  | X | X | X |
|  | Bit5 | SAS detection IPC communication error (* 12 | $0:Not Failure $1:Failure |  | - | - | - |  |  |  | X | X | X |
|  | Bit6 | AEB | $0 :Not Equipped $1: Equipped |  | - | - | - | X | X | X | X | X | X |
|  | Bit7 | (Reserved) | Fixed to 0 |  | - | - | - |  |  |  |  |  |  |
| 49 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |
| 50 |  | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |

X : supported, Space:$FF(Fixed value)

*1 : The acceleration side "+", the deceleration side "-".

*2 : Left turn "-", right turn "+"

*3 : ECU internal control system, encoding : signed data

*4 : Outputs 0xFF sensor calibration not completed at the time.

*5 : Outputs Internal value, sensor calibration not completed at the time.

If SAS communication is disconnected, output is $0000.

*6 Default is "ABS & EBD & ESP ON" state, "ABS & EBD & ESP OFF" state is continued until recover condition is fulfilled after function disable condition fulfilled.

(Function disable condition)

- Other I/O control is executed by Service $ 30

(Recover condition)

- System reset

- IG-OFF

*7 Default is “ESP ON” state, “ESP OFF” state is continued until recover condition is fulfilled after function disable condition fulfilled.

(Function disable condition)

- ESP function is disabled by ESP OFF switch operation

(Recover condition)

- ESP function is enabled by ESP OFF switch operation

- IG-OFF

*8 Default is “ESP ON” state, “ESP OFF” state is continued until recover condition is fulfilled after function disable condition fulfilled.

(Function disable condition)

- ESP function is disabled by Service$30 IOLI$02 ESP function disable command

(Recover condition)

- System reset after Service$30 IOLI$02 ESP function enable command

- IG-OFF

*9 If Steering angle value shows within ± 5 degrees after calibration completed, output is “Neutral”.

If one of the following condition is fulfilled, output is “Not Neutral”.

- Steering angle 0 point value lost

- Sensor calibration not finished

- Steering angle value shows over ± 5 degrees

*10 If one of the following condition is fulfilled, output is “0 (OFF)”.

- Valve failure detected (C1041,C1042,C1043,C1044,C1045,C1046,C1051,C1052,C1053,C1054,C1055,C1056)

- ECU internal failure detected (C1063,C1071)

- Pump failure detected (C1061)

- Power supply voltage abnormal (C1057)

- Brake control is activated (Function control activation)

*11 Output received signal value from SAS. If ESP-SAS communication error is detected, output “0(notESP)”

*12 Output received signal value from SAS. If ESP-SAS communication error is detected, output “0 (No failure)”

*13 not supported MT vehicle.

*14 Supported for K12C/K12M CVT only

*15 only ESP supported

*16 If ECU detect C1082, this value is set to ‘0’. After Variant coding, this value should be set according to Variant information.

*17 not supported nonIS and MT/AMT vehicle.

*18 Output completion status of DTC C101C(Vehicle Variant Data Mismatched) judgment.

RLI　$50-5E

| Position | bit | Parameter Name | CAN ID | min | Max | Factor | Offset | Unit | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Stored Data Table number |  | Fixed to $00 |  |  |  |  | $FF (No stored information) |
| 2 |  | DTC(High byte) |  | - | - | - | - | - |  |
| 3 |  | DTC(Low byte) |  | - | - | - | - | - |  |
| 4 |  | (Reserved) |  | - | - | - | - | - |  |
| 5 |  | The past time from IGON (high byte) |  | 0 | 65535 | 1 | - | s |  |
| 6 |  | The past time from IGON (low byte) |  |  |  |  |  |  |  |
| 7 |  | Master Cylinder Pressure(high byte) | 1AFh | 0 | 200 | 0.003125 | - | Bar |  |
| 8 |  | Master Cylinder Pressure(low byte) |  |  |  |  |  |  |  |
| 9 |  | RF Wheel speed sensor |  | 0 | 255 | 1 | - | Km/h |  |
| 10 |  | LF Wheel speed sensor |  | 0 | 255 | 1 | - | Km/h |  |
| 11 |  | RR Wheel speed sensor |  | 0 | 255 | 1 | - | Km/h |  |
| 12 |  | LR Wheel speed sensor |  | 0 | 255 | 1 | - | Km/h |  |
| 13 |  | Yaw rate sensor learned date(high byte) | 1EFh | -163.84 | 163.83 | 0.005 | -163.84 | deg/s |  |
| 14 |  | Yaw rate sensor learned date(ligh byte) |  |  |  |  |  |  |  |
| 15 |  | Acceleration sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 16 |  | Acceleration sensor signal(low byte) |  |  |  |  |  |  |  |
| 17 |  | Lateral G sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 18 |  | Lateral G sensor signal(low byte) |  |  |  |  |  |  |  |
| 19 |  | Mileage Information(high byte) | 460h | 0 | 1048575 | 1 | - | km |  |
| 20 |  | Mileage Information(middle byte) |  |  |  |  |  |  |  |
| 21 |  | Mileage Information(low byte) |  |  |  |  |  |  |  |
| 22 |  | Accelerator Effective Position | 122h | 0 | 100 | 100/255 | - | % |  |
| 23 |  | Steering angle(High byte) | 119h | -3276.8 | 3276.7 | 0.1 | -3276.8 | deg | Steering Wheel Absolute Angle |
| 24 |  | Steering angle(Low byte) |  |  |  |  |  |  |  |
| 25 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit3 | ID124 Engine Speed Validity | 124h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit4 | ID122 Accelerator Effective Position Validity | 122h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit5 | ID1D1 Brake Stroke Sensor Value Validity | 1D1h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit6 | ID120 Engine Torque Driver Requested Validity | 120h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit7 | ID120 Engine Torque Actual Validity | 120h | $0:Valid $1:Invalid |  | - | - | - |  |
| 26 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit3 | ID$122 Accelerator Actual Position Validity | 122h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit5 | ID460 Mileage Information Validity | 460h | $0:Valid $1:Invalid |  | - | - | - |  |
|  | Bit6 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit7 | ID380 TM Gear Selector Position Validity | 380h | $0:Valid $1:Invalid |  | - | - | - |  |
| 27 |  | Engine RPM (High byte) | 124h | 0 | 16383.75 | 0.25 | - | rpm |  |
| 28 |  | Engine RPM (Low byte) |  |  |  |  |  |  |  |
| 29 |  | Brake Boost Vacuum from ESP | 1AFh | 0 | 127.5 | 0.5 | - | kPa |  |
| 30 |  | TM Gear Selector Position | 380h |  |  |  |  |  |  |
| 31 |  | (Reserved) |  | - | - | - | - | - |  |
| 32 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | ABS active information | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $6=Pre-Active |  | - | - | - | Output ID3A1h State of ABS Function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | EBD active information | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1h State of EBD Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 33 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | Stability Control Function Active information | 3A1h | 0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  | - | - | - | Output ID3A1h State of Stability Control Function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | Traction Control Function Active information | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  | - | - | - | Output ID3A1h State of Traction Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 34 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | HBA Active information | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1h State of Brake Assist function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | Hill Hold Control Active information | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) |  | - | - | - | Output ID3A1h State of Hill Hold Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 35 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit3 | Steering Wheel Absolute Angle Sensor Failure Status | 119h | $0=No Failure $1=Temporarily Failed $2=Permanently Failed |  |  |  |  | Output ID119 Steering Wheel Absolute Angle Sensor Failure Status |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | MSR Active information | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) $5=Disabled (Driver’s Operation) |  | - | - | - | Output ID3A1 State of MSR Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 36 |  | Provide voltage to ECU(high byte) |  | 0 | 1284.486 | 0.0196 | - | v |  |
| 37 |  | Provide voltage to ECU(low byte) |  |  |  |  |  |  |  |
| 38 |  | Motor voltage(high byte) |  | 0 | 1284.486 | 0.0196 | - | v |  |
| 39 |  | Motor voltage(low byte) |  |  |  |  |  |  |  |
| 40 |  | The number of low voltage detect in IGN |  | 0 | 255 | 1 | - | time |  |
| 41 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | ID 119 CS/RC failure status | 119h | $0:no failure $1:failure |  | - | - | - |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit3 | ID 1D0 CS/RC failure status | 1D0 | $0:no failure $1:failure |  | - | - | - |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit5 | ID 124 CS/RC failure status | 124h | $0:no failure $1:failure |  | - | - | - |  |
|  | Bit6 | ID 122 CS/RC failure status | 122h | $0:no failure $1:failure |  | - | - | - |  |
|  | Bit7 | ID 120 CS/RC failure status | 120h | $0:no failure $1:failure |  | - | - | - |  |
| 42 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit5 | ID 0FA CS/RC failure status | 0FAh | $0:no failure $1:failure |  | - | - | - |  |
|  | Bit6 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit7 | ID 1D1 CS/RC failure status | 1D1h | $0:no failure $1:failure |  | - | - | - |  |
| 43 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | ID 1D0 connection status | 1D0h | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit4 | ID 318 connection status | 318h | Fixed to 0 |  | - | - | - |  |
|  | Bit5 | ID 124 connection status | 124h | Fixed to 0 |  | - | - | - |  |
|  | Bit6 | ID 122 connection status | 122h | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit7 | ID 120 connection status | 120h | $0:no timeout $1:timeout |  | - | - | - |  |
| 44 | Bit0 | ID 0FA connection status | 0FAh | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | ID 1D1 connection status | 1D1h | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit4 | ID 119 connection status | 119h | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit5 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit6 | ID 3D0 connection status | 3D0h | $0:no timeout $1:timeout |  | - | - | - |  |
|  | Bit7 | ID 380 connection status | 380h | $0:no timeout $1:timeout |  | - | - | - |  |
| 45 |  | (Reserved) |  | - | - | - | - | - |  |
| 46 |  | IG cycle(high byte) |  | 0 | 65535 | 1 | - | time | If over the 65535 time, fix to 65535 time. |
| 47 |  | IG cycle(low byte) |  |  |  |  |  |  |  |
| 48 |  | The number of BUS OFF in IG line |  | 0 | 255 | 1 | - | time | If over the 255 time, fix to 255 time. |
| 49 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit5 | ID120 Rolling Count Error for 114 | 120h | $0:no Error $1: Error |  |  |  |  |  |
|  | Bit6 | ID119 Rolling Count Error for 3A2 | 119h | $0:no Error $1: Error |  |  |  |  |  |
|  | Bit7 | ID318 Brake Pedal Switch Active | 318h | $0:False $1:True |  |  |  |  |  |
| 50 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | ID119 SteeringWheelAbsoluteAngleSensorFailure Status | 119h | $0=No Failure $1=Temporarily Failed $2=Permanently Failed $3=Reserved |  |  |  |  |  |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 | ID120 Engine Torque Up Failure Status | 120h | $0=Torque Up OK $1=Temporarily Failed $2=Permanently Failed $3=Pending |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |
|  | Bit6 | ID120 Engine Torque Reduction Failure Status | 120h | $0=Torque Reduction OK $1=Temporarily Failed $2=Permanently Failed $3=Pending |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 51 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit4 | ID0FA ACC System Failure Status | 0FAh | $0=No Failure $1=Temporarily Failure $2=Permanently Failure $3=Reserved |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |
|  | Bit6 | ID0FA AEB System Failure Status | 0FAh | $0=No Failure $1=Temporarily Failure $2=Permanently Failure $3=Reserved |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 52 |  | (Reserved) |  | - | - | - | - | - |  |
| 53 |  | (Reserved) |  | - | - | - | - | - |  |
| 54 |  | (Reserved) |  | - | - | - | - | - |  |
| 55 |  | ABS/ESP Variant Data |  | 0 | 255 | 1 |  |  | $FF(No coding) |
| 56 |  | Accelerator Actual Position | 122h | 0 | 100 | 100/255 | - | % |  |
| 57 |  | (Reserved) |  | - | - | - | - | - |  |
| 58 |  | (Reserved) |  | - | - | - | - | - |  |
| 59 |  | (Reserved) |  | - | - | - | - | - |  |
| 60 |  | (Reserved) |  | - | - | - | - | - |  |

RLI　$60-67

| Position | bit | Parameter Name | CAN ID | min | Max | Factor | Offset | Unit | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Stored Data Table number |  | Fixed to $01 |  |  |  |  | $FF (No stored information) |
| 2 |  | (Reserved) |  | - | - | - | - | - |  |
| 3 |  | (Reserved) |  | - | - | - | - | - |  |
| 4 |  | (Reserved) |  | - | - | - | - | - |  |
| 5 |  | The past time from IGON (high byte) |  | 0 | 65535 | 1 | - | s |  |
| 6 |  | The past time from IGON (low byte) |  |  |  |  |  |  |  |
| 7 |  | Master Cylinder Pressure(high byte) | 1AFh | 0 | 200 | 0.003125 | - | Bar |  |
| 8 |  | Master Cylinder Pressure(low byte) |  |  |  |  |  |  |  |
| 9 |  | Vehicle speed(high byte) |  | 0 | 270 | 0.01 |  | Km/h |  |
| 10 |  | Vehicle speed( low byte ) |  |  |  |  |  |  |  |
| 11 |  | Yaw rate sensor learned date(high byte) | 1EFh | -163.84 | 163.83 | 0.005 | -163.84 | deg/s |  |
| 12 |  | Yaw rate sensor learned date(ligh byte) |  |  |  |  |  |  |  |
| 13 |  | Acceleration sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 14 |  | Acceleration sensor signal(low byte) |  |  |  |  |  |  |  |
| 15 |  | Lateral G sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 16 |  | Lateral G sensor signal(low byte) |  |  |  |  |  |  |  |
| 17 |  | Mileage Information(high byte) | 460h | 0 | 1048575 | 1 | - | km |  |
| 18 |  | Mileage Information(middle byte) |  |  |  |  |  |  |  |
| 19 |  | Mileage Information(low byte) |  |  |  |  |  |  |  |
| 20 |  | Accelerator Effective Position | 122h | 0 | 100 | 100/255 | - | % |  |
| 21 |  | Steering angle(High byte) | 119h | -3276.8 | 3276.7 | 0.1 | -3276.8 | deg | Steering Wheel Absolute Angle |
| 22 |  | Steering angle(Low byte) |  |  |  |  |  |  |  |
| 23 | Bit0 | AEB Brake Control Inhibit Status | 2A2h | $0:normal $1:fail |  | - | - | - | Output ID2A2 Brake Control Inhibit Status |
|  | Bit1 |  |  |  |  |  |  |  |  |
|  | Bit2 | State of ABS Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $6=Pre-Active |  |  |  |  | Output ID3A1 State of EBD Function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | State of EBD Function | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1 State of EBD Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 24 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | State of Stability Control Function | 3A1h | 0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  |  |  |  | Output ID3A1 State of Stability Control Function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | State of Traction Control Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  | - | - | - | Output ID3A1 State of Traction Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 25 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | HBA Active information | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1h State of Brake Assist function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | Hill Hold Control Active information | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) |  | - | - | - | Output ID3A1h State of Hill Hold Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 26 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit3 | Acknowledge of Prefill Control Request | 2A2h | $0:False $1:True |  | - | - | - | ID$2A2 Acknowledge of Prefill Control Request |
|  | Bit4 | Acknowledge of AEB Partial Brake Control Request | 2A2h | $0:False $1:True |  | - | - | - | ID$2A2 Acknowledge of AEB Partial Brake Control Request |
|  | Bit5 | Acknowledge of BA Control Request | 2A2h | $0:False $1:True |  | - | - | - | ID$2A2 Acknowledge of BA Control Request |
|  | Bit6 | Acknowledge of AEB Brake Control Request | 2A2h | $0:False $1:True |  | - | - | - | ID$2A2 Acknowledge of AEB Brake Control Request |
|  | Bit7 | Auto Stop Status from ICM | 1D0h | $0:False $1:True |  | - | - | - | ID$1D0 Auto Stop Status from ICM |
| 27 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 28 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 29 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 30 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 31 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 32 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 33 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 34 |  | AEB information | 3A9h |  |  |  |  |  |  |
| 35 |  | (Reserved) |  | - | - | - | - | - |  |
| 36 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | Buzzer Request for AEB | 3A7h | $0=No Request $1=Malfunction $2=Information $3=Warning1 $4=Warning2 |  | - | - | - | ID$3A7 Buzzer Request for AEB |
|  | Bit2 |  |  |  |  |  |  |  |  |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit5 | Buzzer Request for AEB on 3AA | 3AAh | $0=No Request $1=Malfunction $2=Information $3=Warning1 $4=Warning2 |  | - | - | - | ID$3AA Buzzer Request for AEB on 3AA |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 37 |  | (Reserved) |  | - | - | - | - | - |  |
| 38 |  | (Reserved) |  | - | - | - | - | - |  |
| 39 |  | (Reserved) |  | - | - | - | - | - |  |
| 40 |  | (Reserved) |  | - | - | - | - | - |  |
| 41 |  | (Reserved) |  | - | - | - | - | - |  |
| 42 |  | (Reserved) |  | - | - | - | - | - |  |
| 43 |  | (Reserved) |  | - | - | - | - | - |  |
| 44 |  | (Reserved) |  | - | - | - | - | - |  |
| 45 |  | (Reserved) |  | - | - | - | - | - |  |
| 46 |  | (Reserved) |  | - | - | - | - | - |  |
| 47 |  | (Reserved) |  | - | - | - | - | - |  |
| 48 |  | (Reserved) |  | - | - | - | - | - |  |
| 49 |  | (Reserved) |  | - | - | - | - | - |  |
| 50 |  | (Reserved) |  | - | - | - | - | - |  |

RLI　$80-87

| Position | bit | Parameter Name | CAN ID | min | Max | Factor | Offset | Unit | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Stored Data Table number |  | Fixed to $02 |  |  |  |  | $FF (No stored information) |  |
| 2 |  | The past time from IGON (high byte) | - | 0 | 65535 | 1 | - | s |  |  |
| 3 |  | The past time from IGON (low byte) |  |  |  |  |  |  |  |  |
| 4 |  | Master Cylinder Pressure(high byte) | 1AFh | 0 | 200 | 0.003125 | - | Bar |  |  |
| 5 |  | Master Cylinder Pressure(low byte) |  |  |  |  |  |  |  |  |
| 6 |  | Vehicle speed(high byte) | - | 0 | 270 | 0.01 |  | Km/h |  |  |
| 7 |  | Vehicle speed( low byte ) |  |  |  |  |  |  |  |  |
| 8 |  | FR Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |  |
| 9 |  | FL Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |  |
| 10 |  | RR Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |  |
| 11 |  | RL Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |  |
| 12 |  | Yaw rate sensor learned date(high byte) | 1EFh | -163.84 | 163.83 | 0.005 | -163.84 | deg/s |  |  |
| 13 |  | Yaw rate sensor learned date(ligh byte) |  |  |  |  |  |  |  |  |
| 14 |  | Acceleration sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |  |
| 15 |  | Acceleration sensor signal(low byte) |  |  |  |  |  |  |  |  |
| 16 |  | Lateral G sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |  |
| 17 |  | Lateral G sensor signal(low byte) |  |  |  |  |  |  |  |  |
| 18 |  | Mileage Information(high byte) | 460h | 0 | 1048575 | 1 | - | km |  |  |
| 19 |  | Mileage Information(middle byte) |  |  |  |  |  |  |  |  |
| 20 |  | Mileage Information(low byte) |  |  |  |  |  |  |  |  |
| 21 |  | Accelerator Actual Position | 122h | 0 | 100 | 100/255 | - | % |  |  |
| 22 |  | Accelerator Effective Position | 122h | 0 | 100 | 100/255 | - | % |  |  |
| 23 |  | Steering angle(High byte) | 119h | -3276.8 | 3276.7 | 0.1 | -3276.8 | deg | Steering Wheel Absolute Angle |  |
| 24 |  | Steering angle(Low byte) |  |  |  |  |  |  |  |  |
| 25 |  | Engine Torque Actual(high byte) | 120h | -200 | 823.75 | 0.25 | -200 | Nm |  |  |
| 26 |  | Engine Torque Actual(low byte) |  |  |  |  |  |  |  |  |
| 27 |  | Engine Torque Driver Requested(high byte) | 120h | -200 | 823.75 | 0.25 | -200 | Nm |  |  |
| 28 |  | Engine Torque Driver Requested(low byte) |  |  |  |  |  |  |  |  |
| 29 |  | Engine RPM (High byte) | 124h | 0 | 16383.75 | 0.25 | - | rpm | Engine Speed |  |
| 30 |  | Engine RPM (Low byte) |  |  |  |  |  |  |  |  |
| 31 |  | Torque Up Request Value(high byte) | 114h | -200 | 823.75 | 0.25 | -200 | Nm |  |  |
| 32 |  | Torque Up Request Value(low byte) |  |  |  |  |  |  |  |  |
| 33 |  | Fast control torque request value(high byte) | 114h | -200 | 823.75 | 0.25 | -200 | Nm |  |  |
| 34 |  | Fast control torque request value(low byte) |  |  |  |  |  |  |  |  |
| 35 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 36 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 37 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 38 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 39 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 40 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 41 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit2 | State of ABS Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $6=Pre-Active |  |  |  |  | Output ID3A1 State of ABS Function |  |
|  | Bit3 |  |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |  |
|  | Bit5 | State of EBD Function | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1 State of EBD Function |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 42 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit2 | State of Stability Control Function | 3A1h | 0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  |  |  |  | Output ID3A1 State of Stability Control Function |  |
|  | Bit3 |  |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |  |
|  | Bit5 | State of Traction Control Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5=Disabled (Driver’s Operation) |  | - | - | - | Output ID3A1 State of Traction Control Function |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 43 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit2 | State of Brake Assist Function | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1h State of Brake Assist function |  |
|  | Bit3 |  |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |  |
|  | Bit5 | State of MSR Control Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) |  | - | - | - | Output ID3A1h State of MSR Control Function |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 44 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit5 | Torque Intervention Type | 114h | $0=No Intervention $1=Reduction Torque Fast Control $2=Reduction Torque Slow Control $3=Reduction Torque Fast and Slow Control |  | - | - | - |  |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 | Torque Up Request | 114h | $0:False $1:True |  | - | - | - |  |  |
| 45 | Bit0 | Increase/decrease Pressure information(RL) | - | $0=No Control $1=Increase Mode $2=Hold Mode $3=Decrease Mode |  | - | - | - | Not supported |  |
|  | Bit1 |  |  |  |  |  |  |  |  |  |
|  | Bit2 | Increase/decrease Pressure information(RR) | - | $0=No Control $1=Increase Mode $2=Hold Mode $3=Decrease Mode |  | - | - | - | Not supported |  |
|  | Bit3 |  |  |  |  |  |  |  |  |  |
|  | Bit4 | Increase/decrease Pressure information(FL) | - | $0=No Control $1=Increase Mode $2=Hold Mode $3=Decrease Mode |  | - | - | - | Not supported |  |
|  | Bit5 |  |  |  |  |  |  |  |  |  |
|  | Bit6 | Increase/decrease Pressure information(FR) | - | $0=No Control $1=Increase Mode $2=Hold Mode $3=Decrease Mode |  | - | - | - | Not supported |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 46 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit3 | Brake Pedal Switch Active | 318h | $0:False $1:True |  |  |  |  |  |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit5 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit6 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit7 | Auto Stop Status from ICM | 1D0h | $0:False $1:True |  |  |  |  |  |  |
| 47 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit2 | TM Gear Selector Position | 380h | $0=Shift in Progress $1=1st Gear $2=2nd Gear $3=3rd Gear $4=4th Gear $5=Fifth Gear $6-7=Reserved $8=Reverse $9=Neutral $A=Park $B=Drive $C-$D=Reserved $E=Manual Mode $F-$3F=Reserved |  | - | - | - |  |  |
|  | Bit3 |  |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 48 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |  |
|  | Bit4 | TM Actual Gear | 380h | $0=Shift in Progress $1=1st Gear $2=2nd Gear $3=3rd Gear $4=4th Gear $5=Fifth Gear $6-7=Reserved $8=Reverse $9=Neutral $A=Reserved $B=Drive $C-$F=Reserved |  |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |  |
|  | Bit6 |  |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |  |
| 49 |  | (Reserved) |  | - | - | - | - | - |  |  |
| 50 |  | (Reserved) |  | - | - | - | - | - |  |  |

RLI　$88-8B

| Position | bit | Parameter Name | CAN ID | min | Max | Factor | Offset | Unit | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Stored Data Table number |  | Fixed to $03 |  |  |  |  | $FF (No stored information) |
| 2 |  | The past time from IGON (high byte) | - | 0 | 65535 | 1 | - | s |  |
| 3 |  | The past time from IGON (low byte) |  |  |  |  |  |  |  |
| 4 |  | Master Cylinder Pressure(high byte) | 1AFh | 0 | 200 | 0.003125 | - | Bar |  |
| 5 |  | Master Cylinder Pressure(low byte) |  |  |  |  |  |  |  |
| 6 |  | Vehicle speed(high byte) | - | 0 | 270 | 0.01 |  | Km/h |  |
| 7 |  | Vehicle speed( low byte ) |  |  |  |  |  |  |  |
| 8 |  | FR Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |
| 9 |  | FL Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |
| 10 |  | RR Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |
| 11 |  | RL Wheel speed sensor | 1B8h | 0 | 255 | 1 | - | Km/h |  |
| 12 |  | Yaw rate sensor learned date(high byte) | 1EFh | -163.84 | 163.83 | 0.005 | -163.84 | deg/s |  |
| 13 |  | Yaw rate sensor learned date(ligh byte) |  |  |  |  |  |  |  |
| 14 |  | Acceleration sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 15 |  | Acceleration sensor signal(low byte) |  |  |  |  |  |  |  |
| 16 |  | Lateral G sensor signal(high byte) | 1EFh | -17.625 | 17.625 | 0.0625 | -17.625 | m/s2 |  |
| 17 |  | Lateral G sensor signal(low byte) |  |  |  |  |  |  |  |
| 18 |  | Mileage Information(high byte) | 460h | 0 | 1048575 | 1 | - | km |  |
| 19 |  | Mileage Information(middle byte) |  |  |  |  |  |  |  |
| 20 |  | Mileage Information(low byte) |  |  |  |  |  |  |  |
| 21 |  | Accelerator Actual Position | 122h | 0 | 100 | 100/255 | - | % |  |
| 22 |  | Accelerator Effective Position | 122h | 0 | 100 | 100/255 | - | % |  |
| 23 |  | Steering angle(High byte) | 119h | -3276.8 | 3276.7 | 0.1 | -3276.8 | deg | Steering Wheel Absolute Angle |
| 24 |  | Steering angle(Low byte) |  |  |  |  |  |  |  |
| 25 |  | Engine Torque Actual(high byte) | 120h | -200 | 823.75 | 0.25 | -200 | Nm |  |
| 26 |  | Engine Torque Actual(low byte) |  |  |  |  |  |  |  |
| 27 |  | Engine Torque Driver Requested(high byte) | 120h | -200 | 823.75 | 0.25 | -200 | Nm |  |
| 28 |  | Engine Torque Driver Requested(low byte) |  |  |  |  |  |  |  |
| 29 |  | Engine RPM (High byte) | 124h | 0 | 16383.75 | 0.25 | - | rpm | Engine Speed |
| 30 |  | Engine RPM (Low byte) |  |  |  |  |  |  |  |
| 31 |  | Brake Boost Vacuum from ESP | 1AFh | 0 | 127.5 | 0.5 | - | kPa |  |
| 32 |  | Barometric Pressure | 32Eh | 0 | 255 | 1 | - | kPa |  |
| 33 |  | (Reserved) |  | - | - | - | - | - |  |
| 34 |  | (Reserved) |  | - | - | - | - | - |  |
| 35 |  | (Reserved) |  | - | - | - | - | - |  |
| 36 |  | (Reserved) |  | - | - | - | - | - |  |
| 37 |  | (Reserved) |  | - | - | - | - | - |  |
| 38 |  | (Reserved) |  | - | - | - | - | - |  |
| 39 |  | (Reserved) |  | - | - | - | - | - |  |
| 40 |  | (Reserved) |  | - | - | - | - | - |  |
| 41 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | State of ABS Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $6=Pre-Active |  |  |  |  | Output ID3A1 State of ABS Function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | State of EBD Function | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1 State of EBD Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 42 |  | (Reserved) |  | - | - | - | - | - |  |
| 43 | Bit0 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit1 | ABS ESP Motor Control Active | 3A1h | $0:False $1:True |  | - | - | - |  |
|  | Bit2 | State of Brake Assist Function | 3A1h | $0=Enabled $1=Active $2=Disabled(Failed) $3=Disabled(Maintenance) $4=Disabled(Reversible) |  | - | - | - | Output ID3A1h State of Brake Assist function |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 | Hill Hold Control Function | 3A1h | $0=Enabled $1=Active $2=Disabled (Failed) $3=Disabled (Maintenance) $4=Disabled (Reversible) $5-7=Reserved |  | - | - | - | Output ID3A1h State of Hill Hold Control Function |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 44 |  | (Reserved) |  | - | - | - | - | - |  |
| 45 |  | (Reserved) |  | - | - | - | - | - |  |
| 46 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | ESP Check Request | 1D0h | $0:False $1:True |  |  |  |  |  |
|  | Bit3 | Brake Pedal Switch Active | 318h | $0:False $1:True |  |  |  |  |  |
|  | Bit4 | On Idle Stop Status | 1D0h | $0=Engine Running $1=Idle Stop $2=Reserved $3=Engine Stall Fixed to 0 |  | - | - | - |  |
|  | Bit5 |  |  |  |  |  |  |  |  |
|  | Bit6 | Engine Stop Request from ICM | 1D0h | $0:False $1:True |  |  |  |  |  |
|  | Bit7 | Auto Stop Status from ICM | 1D0h | $0:False $1:True |  |  |  |  |  |
| 47 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | TM Gear Selector Position | 380h | $0=Shift in Progress $1=1st Gear $2=2nd Gear $3=3rd Gear $4=4th Gear $5=Fifth Gear $6-7=Reserved $8=Reverse $9=Neutral $A=Park $B=Drive $C-$D=Reserved $E=Manual Mode $F-$3F=Reserved |  | - | - | - |  |
|  | Bit3 |  |  |  |  |  |  |  |  |
|  | Bit4 |  |  |  |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 48 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  | - | - | - |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit4 | TM Actual Gear | 380h | $0=Shift in Progress $1=1st Gear $2=2nd Gear $3=3rd Gear $4=4th Gear $5=Fifth Gear $6-7=Reserved $8=Reverse $9=Neutral $A=Reserved $B=Drive $C-$F=Reserved |  |  |  |  |  |
|  | Bit5 |  |  |  |  |  |  |  |  |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 49 | Bit0 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit1 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit2 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit3 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit4 | (Reserved) |  | Fixed to 0 |  |  |  |  |  |
|  | Bit5 | Engine Control Status for Battery Control | 2E4h | $0=Engine Stall $1=Idling Stop + Coast Stop $2=Stop Preparation $3=Engine Start (Only Pb) $4=Engine Start (Pb + Li) $5=Engine Running $6=Assist $7=Regeneration |  | - | - | - | Not support |
|  | Bit6 |  |  |  |  |  |  |  |  |
|  | Bit7 |  |  |  |  |  |  |  |  |
| 50 |  | (Reserved) |  | - | - | - | - | - |  |

DTC List

| DTCs | Contents | ABS |  |  | ESP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MT |  | A T | MT | AT |  |
|  |  | With IS | With out IS |  |  | With IS | With out IS |
| C1015 | Longitudinal G sensor failure |  |  |  | X | X | X |
| C1016 | Brake lamp SW failure | X | X | X | X | X | X |
| C1017 | Lateral G sensor failure |  |  |  | X | X | X |
| C101C | Vehicle Variant Data Mismatched (*5 |  |  |  | X | X | X |
| C1020 | The master cylinder pressure sensor system power |  |  |  | X | X | X |
| C1021 | Wheel speed sensor failure / Open/Short (FR) | X | X | X | X | X | X |
| C1022 | Wheel speed sensor abnormality detection software (FR) | X | X | X | X | X | X |
| C1023 | Yaw rate sensor system failure |  |  |  | X | X | X |
| C1024 | Steering angle sensor failure |  |  |  | X | X | X |
| C1025 | Wheel speed sensor failure, Open/Short (FL) | X | X | X | X | X | X |
| C1026 | Wheel speed sensor failure software (FL) | X | X | X | X | X | X |
| C1027 | EPS OFF SW failure |  |  |  | X | X | X |
| C1028 | The master cylinder pressure sensor failure |  |  |  | X | X | X |
| C102B | Vacuum sensor failure (Support only ISG Vehicle) |  |  |  |  | X | X |
| C1031 | Wheel speed sensor failure, Open/Short (RR) | X | X | X | X | X | X |
| C1032 | Wheel speed sensor abnormality detection software (RR) | X | X | X | X | X | X |
| C1035 | Wheel speed sensor failure, Open/Short (RL) | X | X | X | X | X | X |
| C1036 | Wheel speed sensor failure software (RL) | X | X | X | X | X | X |
| C1037 | Steering angle sensor system power failure (*3 |  |  |  | X | X | X |
| C1038 | Steering angle sensor detection ABS rolling counter error |  |  |  | X | X | X |
| C1040 | ESP continuous operation failure |  |  |  | X | X | X |
| C1041 | Holding valve solenoid failure (FR) | X | X | X | X | X | X |
| C1042 | Pressure reducing valve solenoid failure (FR) | X | X | X | X | X | X |
| C1043 | Switching solenoid valve failure (solenoid1) |  |  |  | X | X | X |
| C1044 | Switching solenoid valve failure (solenoid2) |  |  |  | X | X | X |
| C1045 | Holding valve solenoid failure (FL) | X | X | X | X | X | X |
| C1046 | Pressure reducing valve solenoid failure (FL) | X | X | X | X | X | X |
| C104F | Vacuum SW failure(*6 | X |  | X |  |  |  |
| C1051 | Holding valve solenoid failure (RR) | X | X | X | X | X | X |
| C1052 | Pressure reducing valve solenoid failure (RR) | X | X | X | X | X | X |
| C1053 | Accumulator solenoid valve failure (solenoid1) |  |  |  | X | X | X |
| C1054 | Accumulator solenoid valve failure (solenoid2) |  |  |  | X | X | X |
| C1055 | Holding valve solenoid failure (RL) | X | X | X | X | X | X |
| C1056 | Pressure reducing valve solenoid failure (RL) | X | X | X | X | X | X |
| C1057 | Abnormal power supply voltage | X | X | X | X | X | X |
| C1061 | Pump motor relay failure | X | X | X | X | X | X |
| C1063 | Fail-safe relay failure | X | X | X | X | X | X |
| C1065 | Brake lamp relay drive failure |  |  |  | X | X | X |
| C1069 | SAS specification undetermined(*5 |  |  |  | X | X | X |
| C1071 | ECU internal failure | X | X | X | X | X | X |
| C1074 | Longitudinal G sensor Calibration not completed (*1 |  |  |  | X | X | X |
| C1075 | Steering angle sensor calibration not completed (*1 |  |  |  | X | X | X |
| C1076 | The master cylinder pressure sensor calibration not completed(*1 |  |  |  | X | X | X |
| C1078 | Lateral G sensor calibration not completed (*1 |  |  |  | X | X | X |
| C107F | Air bleed drive not completed (*7 | X | X | X | X | X | X |
| C1080 | Vehicle spec not detected (*9 | X | X | X |  |  |  |
| C1081 | Fill and bleed not completed (*2 | X | X | X | X | X | X |
| C1082 | ABS / ESP variant data undefined (*3 |  |  |  | X | X | X |
| C1083 | ABS/ESP variant out of undefined (*4 |  |  |  | X | X | X |
| C1090 | ECM detection ABS rolling counter failure |  |  |  | X | X | X |
| C1091 | CAN invalid data from ECM(*5 | X | X | X | X | X | X |
| C1092 | CAN invalid data from TCM (*8 |  |  |  |  | X | X |
| C1094 | Torque Request Reject Status (Com. Error)(*5 |  |  |  | X | X | X |
| C1095 | CAN invalid data from HCU |  |  |  |  |  |  |
| C1099 | CAN Invalid data from ICM |  |  |  |  | X |  |
| C109D | CAN invalid data from BMS ‘A’ (12VLi) |  |  |  |  |  |  |
| C10A0 | ABS / ESP detection AEB rolling counter failure |  |  |  |  | X | X |
| C10A1 | ABS / ESP detection AEB checksum error |  |  |  |  | X | X |
| C10A3 | AEB continuous operation request failure |  |  |  |  | X | X |
| C10A7 | ABS / ESP detection MFL rolling counter failure |  |  |  | X |  | X |
| C10A8 | ABS / ESP detection MFL checksum error |  |  |  | X |  | X |
| C10A9 | MFL continuous operation request failure |  |  |  | X |  | X |
| U0073 | Control Module Communication Bus Off | X | X | X | X | X | X |
| U0100 | Lost Communication With ECM/PCM | X | X | X | X | X | X |
| U0101 | Lost Communication With TCM (*8 |  |  |  |  | X | X |
| U0126 | Lost Communication With Steering Angle Sensor Module |  |  |  | X | X | X |
| U0140 | Lost Communication With BCM |  |  |  | X | X | X |
| U0155 | Lost Communication With Instrument Panel Cluster(IPC) Control Module | X | X | X | X | X | X |
| U1082 | Lost Communication With ICM |  |  |  |  | X |  |
| U108A | Lost Communication With AEB |  |  |  |  | X | X |
| U108E | Lost Communication With BMS ‘A’ (12VLi) |  |  |  |  |  |  |
| U1093 | Lost Communication With MFL |  |  |  | X |  | X |
| U0293 | Lost Communication With HCU |  |  |  |  |  |  |
| C1802 | Control Module Programming Error (*5 | X | X | X | X | X | X |

X : supported

*1 When ECU is delivered to the customer, this DTC shall be set as default. If the sensor calibration is  successfully finished by manual operation or Service $30 (ESP only car), this shall be cleared automatically (i.e. there is no case of that the status is past).

*2 This DTC shall be applied only for HCU DRY parts, and be set as default at the delivery. If the writing of fill and bleed completion flag is successfully done by Service $3B, it shall be automatically cleared (i.e. there is no case of that the status is past). But ECU shall disable all the control functions and turn the warning lamps until a reset (IGN OFF/ON, system reset).

This DTC shall be set until fill and bleed completion flag is set. If this flag is set at least once, this DTC shall never be detected. And during detecting this DTC, ECU should not detect any failure except related to Fill and Bleed. And this state should be kept till next system reset.

*3 If the variant coding is successfully finished by Service $3B, this DTC shall be automatically cleared (i.e. there is no case of that the status is past).

*4 If the variant data is cleared by Service $3B (=$FF is written), this DTC shall be automatically cleared (i.e. there is no case of that the status is past).

*5 If ECU judged to return normal, this DTC shall be automatically cleared(i.e. there is no case of that the status is past). Regarding to C1069, please refer to CAN specification.

*6 Supported for K12C/K12M CVT only

*7 If ECU starts executing fill and bleed drive or Air Bleeding drive(secondary/thirdly), this DTC should be set. And then if ECU finishes executing motor drive normally after Fill and Bleed or Air Bleeding drive(secondary/thirdly), this DTC shall be automatically cleared(i.e. there is no case of that the status is past).

*8 Supported for AT/CVT/AMT only

*9 This DTC shall be automatically cleared on every IGN timinmg(i.e. there is no case of that the status is past).

Freeze Frame Data Specification

【Target Vehicle】

All vehicles

【Function Overview】

Store the vehicle information at the time of failure detection.

【Stored failure】

All the failures except the following failures.

- U0073 Control Module Communication Bus Off

- C1069 SAS specification undetermined

- C1074 Longitudinal G sensor calibration not completed

- C1075 Steering angle sensor calibration not completed

- C1076 Master cylinder pressure sensor calibration not completed

- C1078 Lateral G sensor calibration not completed

- C107F Air bleed drive not completed

- C1081 Fill and bleed not completed

【Stored Timing】

When the failure is detected

【Updated Timing】

The DTC can be stored at most 15(In case of ABS, 10), and 2 DTCs for the first stored are locked.

If 16th DTC is stored, 1st DTC is overwritten by 16th DTC(In case of ABS, 11th DTC).

【Stored Vehicle Information】

Refer to “8	RLI stored Data Table”

RLI　$50-5E

- Passed time after IG on

- Master cylinder pressure

- Each wheel speed

- Yaw rate learning value

- Longitudinal G sensor value

- Lateral G sensor value

- Odometer information (ID460h Mileage Information)

- Accelerator position information (ID122h Accelerator Effective Position)

Accelerator position information (ID122h Accelerator Actual Position)

- Steering angle sensor value (ID119h Steering Wheel Absolute Angle)

- Validity information

ID120h Engine Torque Actual Validity / Engine Torque Driver Requested Validity

ID122h Accelerator Actual Position Validity

ID122h Accelerator Effective Position Validity

ID124h Engine Speed Validity

ID1D1h Brake Stoke Sensor Value Validity

ID380h TM Gear Selector Position Validity

ID460h Mileage Information Validity

- Engine speed

- Vacuum sensor value

- Gear information (ID380h TM Gear Selector Position)

- EBD function information (ID3A1h State of EBD Function)

- ABS function information (ID3A1h State of ABS Function)

- Traction control function information (ID3A1h State of Traction Control Function)

- Stability control function information (ID3A1h State of Stability Control Function)

- Hill hold control function information (ID3A1h State of Hill Hold Control Function)

- HBA function information (ID3A1h State of HBA Function)

- MSR function information (ID3A1h State of MSR Function)

- Steering angle sensor status signal (ID119h Steering Wheel Absolute Angle Sensor Failure Status)

- Supply voltage to the ECU

- Motor battery voltage

- Count of detected low voltage in IG cycle

- The target CAN ID at the time of CS/RC error detection (ID120/122/124/1D0/1D1/119/0FAh)

- The target CAN ID at the time of error detection (ID120/122/124/318/1D0/1D1/380/3D0/119/0FAh)

- Number of IG cycle

ab)	Count of detected bus off in IG cycle

ac) Failure Status Signal

(ID120h Engine Torque Reduction Failure Status / Engine Torque Up Failure Status

ID119h Steering Wheel Absolute Angle Sensor Failure Status

ID0FAh AEB System Failure Status / ACC System Failure Status)

ad) Brake SW information (ID318h Brake Pedal Switch Active)

※Make it possible to read the data with SUZUKI Tool.

Drive recorder Specification

【Function Overview】

Store the vehicle information at the following control function is active.

- AEB

- Prefill

- ABS

- ESP

- Brake assist function for insufficient booster vacuum during engine stop

These functions have the data area for each function.

【Stored Timing】

ESC should store on the following timing.

10

～500ms

Function

Function

500ms

500ms

Before 1000ms, before 500ms, active timing, after 500ms.

ECU cannot store the data for two more function at the same time. So the data of function that the timing is the fastest is stored. If some functions start at the same time, ECU should store as below.

Prefill > ESP > ABS > Brake assist function for insufficient booster vacuum during engine stop.

Generally AEB start after prefill control. So ESC cannot store AEB data. Therefore in case of AEB, ECU should store the data, even if other function is active.

【Active timing】

The Active timing is below.

- AEB

The one of the following signal is set from $0 to $1.And the trigger signal should be stored as $1.

Acknowledge of AEB Brake Control Request(CAN ID: 2A2h)

Acknowledge of AEB Partial Brake Control Request(CAN ID: 2A2h)

Acknowledge of AEB Prefill Control Request(CAN ID: 2A2h)

Acknowledge of BA Control Request(CAN ID: 2A2h)

- Prefill

The following signal is set from $0 to $1.And the trigger signal should be stored as $1.

Acknowledge of AEB Prefill Control Request(CAN ID: 2A2h)

- ABS

The following signals are set from $0 to $1.And the trigger signal should be stored as $1.

State of ABS Function(CAN ID: 3A1h)

- ESP

The following signals are set from $0 to $1.And the trigger signal should be stored as $1.

State of Stability Control Function(CAN ID: 3A1h)

- Brake assist function for insufficient booster vacuum during engine stop

ECU recognize this function in control.

【Updated Timing】

If any function is in control more than once, ECU should store the latest information.

ECU should not store the any information for 1.6sec from end of control timing of any function.

【Stored Vehicle Information】

Refer to “8	RLI stored Data Table”

Flag – DTC Transition Table

【ABS】

<Fill and Bleed>

|  | IGN ON | After Fill and Bleed execution requested | After Fluid Filling flag set requested | After Fill and Bleed completion flag set requested | IGN OFF → IGN ON |
| --- | --- | --- | --- | --- | --- |
| Fluid Filling flag | 0 | 0 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 0 | 1 | 1 | 1 | 1 |
| Fill and Bleed completion flag | 0 | 0 | 0 | 1 | 1 |
| BRAKE W/L | ○ | ○ | ○ | ○ | - |
| ABS W/L | △ | △ | △ | ○ | - |
| C1081 | Stored | Stored | Stored | Not stored | Not stored |
| C107F | Not stored | Stored | Stored | Not stored | Not stored |

<Air Bleeding>

|  | IGN ON | After Air Bleeding execution requested | After Air Bleeding completion requested | IGN OFF → IGN ON |
| --- | --- | --- | --- | --- |
| Fluid Filling flag | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 1 | 1 | 1 | 1 |
| Fill and Bleed completion flag | 1 | 1 | 1 | 1 |
| BRAKE W/L | - | - | - | - |
| ABS W/L | - | △ | - | - |
| C1081 | Not stored | Not stored | Not stored | Not stored |
| C107F | Not stored | Stored | Not stored | Not stored |

<WET parts>

|  | IGN ON | Decrease pressure drive execution | Decrease pressure drive complete | IGN OFF → IGN ON |
| --- | --- | --- | --- | --- |
| Fluid Filling flag | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 0 | 0 | 1 | 1 |
| Fill and Bleed completion flag | 0 | 0 | 1 | 1 |
| BRAKE W/L | ○ | ○ | ○ | - |
| ABS W/L | △ | △ | ○ | - |
| C1081 | Not stored | Not stored | Not stored | Not stored |
| C107F | Not stored | Not Stored | Not stored | Not stored |

【ESP】

<Fill and Bleed → Sensor Calibration>

|  | Fill and Bleed |  |  |  |  | Variant Coding |  | Sensor Calibration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IGN ON | After Fill and Bleed execution requested | After Fluid Filling flag set requested | After Fill and Bleed completion flag set requested | IGN OFF → IGN ON | Before Variant Coding | After Variant Coding | Before Sensor Calibration | After Sensor Calibration |
| Fluid Filling flag | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Fill and Bleed completion flag | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| BRAKE W/L | ○ | ○ | ○ | ○ | - | - | - | - | - |
| ABS W/L | △ | △ | △ | ○ | ○ | ○ | - | - | - |
| ESP W/L | ○ | ○ | ○ | ○ | ○ | ○ | ○ | ○ | - |
| ESP OFF/L | ○ | ○ | ○ | ○ | ○ | ○ | - | - | - |
| C1081 | Stored | Stored | Stored | Not stored | Not stored | Not stored | Not stored | Not stored | Not stored |
| C1082 | Stored | Stored | Stored | Stored | Stored | Stored | Not stored | Not stored | Not stored |
| C1074/75/76/78 | Stored | Stored | Stored | Stored | Stored | Stored | Stored | Stored | Not stored |
| C107F | Not stored | Stored | Stored | Not stored | Not stored | Not stored | Not stored | Not stored | Not stored |

<Air Bleeding> ※In case of that it is performed after the sensor calibration.

|  | IGN ON | After Air Bleeding execution requested | After Air Bleeding completion requested | IGN OFF → IGN ON |
| --- | --- | --- | --- | --- |
| Fluid Filling flag | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 1 | 1 | 1 | 1 |
| Fill and Bleed completion flag | 1 | 1 | 1 | 1 |
| BRAKE W/L | - | - | - | - |
| ABS W/L | - | △ | ○ | - |
| ESP W/L | - | ○ | ○ | - |
| ESP OFF/L | - | ○ | ○ | - |
| C1081 | Not stored | Not stored | Not stored | Not stored |
| C1082 | Not stored | Not stored | Not stored | Not stored |
| C1074/75/76/78 | Not stored | Not stored | Not stored | Not stored |
| C107F | Not stored | Stored | Not stored | Not stored |

<WET Supplies>

|  | IGN ON | Decrease pressure drive execution | Decrease pressure drive complete | IGN OFF → IGN ON | After Sensor Calibration |
| --- | --- | --- | --- | --- | --- |
| Fluid Filling flag | 1 | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 0 | 0 | 1 | 1 | 1 |
| Fill and Bleed completion flag | 0 | 0 | 1 | 1 | 1 |
| BRAKE W/L | ○ | ○ | ○ | - | - |
| ABS W/L | △ | △ | ○ | - | - |
| ESP W/L | ○ | ○ | ○ | ○ | - |
| ESP OFF/L | ○ | ○ | ○ | - | - |
| C1081 | Not stored | Not stored | Not stored | Not stored | Not stored |
| C1082 | Not stored | Not stored | Not stored | Not stored | Not stored |
| C1074/75/76/78 | Stored | Stored | Stored | Stored | Not stored |
| C107F | Not stored | Not stored | Not stored | Not stored | Not stored |

Enable to support in both

or

|  | After the sensor calibration | Decompression drive execution | Decompression drive completion | IGN OFF → IGN ON |
| --- | --- | --- | --- | --- |
| Fluid Filling flag | 1 | 1 | 1 | 1 |
| Fill and Bleed execution flag | 0 | 0 | 1 | 1 |
| Fill and Bleed completion flag | 0 | 0 | 1 | 1 |
| BRAKE W/L | ○ | ○ | ○ | - |
| ABS W/L | △ | △ | ○ | - |
| ESP W/L | ○ | ○ | ○ | - |
| ESP OFF/L | ○ | ○ | ○ | - |
| C1081 | Not stored | Not stored | Not stored | Not stored |
| C1082 | Not stored | Not stored | Not stored | Not stored |
| C1074/75/76/78 | Not stored | Not stored | Not stored | Not stored |
| C107F | Not stored | Not stored | Not stored | Not stored |

Notes

<Lamp Status>

○ : On   - : Off   △ : Blinking (1Hz)

※	After Fill and Bleed completion flag set requested, Air Bleeding completion requested, and Decrease Pressure drive completion: In the state between Output control performed and communication end or IGN OFF.

※	This is on the assumption that there is no failure except for C1074 and C1081. (DRY parts are out of considering.)

※	If ECU has never executed Fill and Bleed, ECU shall not detect except the following DTCs.
<ESP> 
C1020, C1028, C1041, C1042, C1043, C1044, C1045, C1046, C1051, C1052, 
C1053, C1054, C1055, C1056, C1057, C1061, C1063, C1071, C1074, C1075, 
C1076, C1078, C107F, C1082, C1083 
U0073
<ABS>
C1041, C1042, C1045, C1046, C1051, C1052, C1055, C1056, C1061, C1063, C1071, C1057, C107F, C1081

Variant Coding Specification

This specification is applicable only for ESP system.

【Variant Coding Flow】

<Diagnostic command>

C1082 is active

Repeat Variant coding flow after checking state of that variant coding has not been done yet

All $FF

(Variant coding has not been done yet)

C1082 is active

C1083 is active or C1082 is not active

Motor, SOL/V, ECU internal failure is active

Receive the variant code which is out of spec

POWER OFF (IGN OFF)

POWER OFF (IGN OFF)

POWER OFF (IGN OFF)

POWER ON (IGN ON)

C1082 is not active

Repeat again after the variant erasing

- Need to replace H/U

- Check tester (or PC)

Complete

Complete

Complete

Make a reset just after the variant coding/erasing

-> Refer to above chapter

Variant Code : 1byte

Password : 2byte

SUZUKI

Parts Number

Verification

Variant Data

Verification

Read DTC

Variant coding ($XX)

Variant coding ($FF)

Release Security

Release Security

SUZUKI

Parts Number

Verification

Read DTC

BASE

Parts Number

Verification

Notes

- Regardless of the status of the following flags, the variant coding shall be enabled.

- Fill and bleed completion flag

- Fill and bleed execution flag

- Fluid-filling flag

- The variant coding/erasing shall not be enabled if ESP detects the hardware failure.

- About each diagnostic command, refer to each chapter above.

- The variant coding/erasing shall be accepted only in the state of the security released.

(ECU shall send the negative response to the request in the state of the security locked.)

About the way to change the security status, refer to Security Access above.

- If the variant coding/erasing is performed, ECU shall make a reset after sending the positive response.

- If the variant erasing is requested to ECU which has not been written the variant data (=SUZUKI parts number: all $FF), ECU shall send the positive response.

- ECU shall send the negative response to the request to write an undefined variant.

- The acceptable variants shall be limited depended on each BASE parts number.

【Lamp spec and DTC handling】

- In case of that Fill and Bleed has not been completed

(Precondition: C1081 is active and Fill and Bleed completion flag = 0)

|  | Before the variant coding | After the variant coding |
| --- | --- | --- |
| ESP W/L | On | On |
| ESP OFF/L | On | On |
| ABS W/L | Blinking (1Hz) | Blinking (1Hz) |
| BRAKE W/L | On | On |
| C1081 | Stored | Stored |
| C1082 | Stored | Not stored |
| C1074 | Stored | Stored |
| C1075/76/78 | Stored | Stored |

- In case of that Fill and Bleed has been completed

|  | Before the variant coding | After the variant coding |
| --- | --- | --- |
| ESP W/L | On | On |
| ESP OFF/L | On | On |
| ABS W/L | On | Off |
| BRAKE W/L | Off | Off |
| C1081 | Not stored | Not stored |
| C1082 | Stored | Not stored |
| C1074 | Stored | Stored |
| C1075/76/78 | Stored | Stored |

Variant Coding Mismatch Detection Specification

【Target Vehicle】

This specification is applicable only for ESP system.

【Function Overview】

Compare the vehicle specification based on the written variant code and the vehicle specification judged from the CAN signal. In case of mismatch, DTC is stored.

【Stored failure】

C101C Vehicle Variant Data Mismatched

* If IG-OFF, this DTC shall be automatically cleared(i.e. there is no case of that the status is past).

【Variant Coding Mismatch Detection Conditions】

- In case of Transmission Conditions

| Judgment condition | TM Type from IPC |
| --- | --- |
| 4AT | $2 |
| MT | $0 |

* It does not judge other than above.

- In case of Idle stop Conditions

| Judgment condition | IS Equipped |
| --- | --- |
| With idle stop | $1 |
| With Out idle stop | $0 |

* It does not judge other than above.

CHANGE HISTORY

| Index | Date | Chapter | Contents |
| --- | --- | --- | --- |
| 00 |  | All | Newly created Based on YS Series |
| 01 |  | 7.5 | Support RLI$30, 31 |
| 01 |  | 8 | Add to *5) of RLI $00 Steering angle values(Position 25 and 26) |
| 01 |  | 8 | Add to Bit0 of Position 39 of RLI $00 Variant Coding Mismatch Detection status |